Chapter 2 (Sherman Notes)  Updated 9/16/15

1. Pitch Static Stability

The most common models for the lift and drag forces on an airfoil are:

CL=CL0+CLαα	(1)

CD=CDmin + KCL2.

The key variable in (1) is the angle of attack, α. Let xcgdenote the distance from the wing leading edge to the plane center of gravity (cg), and let xacdenote the distance from the wing leading edge to the plane areodynamic center (ac),  Let h=xcgcand hn=xacc denote the scaled center of gravity and areodynamic center, respectively, where cis the airfoil mean chord length. Note that h may be in relation to the airfoil itself, or to the vehicle to which it is attached. In these note, unless otherwise stated, it will denote the distance from the nose of the airplane to the plane’s cg. Then the airfoil moment (scaled by QS {c) is:

Cm=Cmac+CL(h−hn)=Cmac+ (CL0+CLαα)(h−hn)=[Cmac+ CL0(h−hn)]+[CLα(h−hn)]α=ΔCm0+Cmαα. (2)

A key parameter in (2) that controls the longitudinal stability of the plane is the airfoil pitch moment derivative: Cmα=dCmdα=CLα(h−hn). It should be noted that, since we require that CLα>0, it follows that in order for the wing to be a stabilizing component of the plane (i.e. Cmα<0. See Example 1 below), it must satisfy h<hn. Typically this condition does not hold. In other words, typically the wing is a destabilizing component.

Example 1. Consider three planes have plots of Cm(α)shown in Figure 1.

Figure 1. Plots of Cm(α) for airplanes #1, #2, and #3.

Plane #1 has Cmα>0, and so a positive perturbation in the pitch angle, α , results in a positive pitching moment, thereby causing α to move further away from equilibrium. Hence, plane #1 is longitudinally unstable. Plane #2 has Cm<0, and so one might suspect it is longitudinally stable. However, the equilibrium point [i.e. the value of α0 such that Cm(α0)=0] is negative. This author is not aware of a single plane whose wing is pitched downward. Hence, while, theoretically, plane #2 is longitudinally stable, such a plane probably does not exist. Plane #3 has both Cmα<0 and α0>0. Hence, it is a longitudinally stable real plane. □

Definition 1 For an airplane in a longitudinally balanced (or equilibrium) condition, with corresponding angle of attack, αo, (where we will assume only the case αo≥0), suppose that the airplane angle of attack is changed to a value α=αo+Δα. If the resulting non-zero pitch moment, Cm(α)acts in a manner to tend to bring the angle of attack back towardαo, then the airplane is said to have positive pitch stiffness. If this moment acts in a manner to drive the angle of attack further away fromαo, then the airplane is said to have negative pitch stiffness. Mathematically, this is equivalent to the condition that the pitch moment derivative satisfy∂Cm(α)∂α|α=α0<0. Finally, if ∂Cm(α)∂α|α=α0=0, then the airplane is said to have zero pitch stiffness (or, the plane is said to have neutral longitudinal stability.)

The two main components of an airplane that contribute to its stability are the wing and the tail. Each

\component has exactly the same lift and moment behavior as that described by (1) and (2), respectively. In the following two sections the details of these expressions will be summarized.

2.1 The Wing (or Wing/Body)

The following equations are for the pitch moment coefficients of the wing.

CLw=CL0w+ CLαwα	(3)  (2.7)

Cmw=Cm0w+Cmαwα	(4) Fig.2.6

where                                       Cm0w=Cmacw+CL0w (h−hnw) 	(5)  (2.8)

and                                                 Cmαw=CLαw (h−hnw) .	(6)  (2.9)

The same equations hold for the wing/body; in which case the subscript ‘w’ is replaced by the subscript ‘wb’. In this case, if only the wing(tail)-alone lift derivatives are given, then they must be corrected to account for a finite aspect ratio. This correction is: CLαwb=CLαw1+CLαw(π AR).

2.2 The Tail

CLt=CLαtαt=CLαt[α−(iw−it)−ε]	(7)

where the variables in (7) are defined in Figure 2.7 on p.45 and in Figure 2.9 on p.47.

Cmt=Cm0t+Cmαtα	(8)

where                                                Cm0t=ηVHCLαt[ε0+(iw−it)]	(9)

and                                                       Cmαt=−ηVHCLαt1−dεdα.	(10)

We have:            VH=ltStcS,  ε0=2CLw(α=α0)π ARw,    η=ρVt22ρVw22=QtQw  ,   dεdα|α0=2CLαwπ ARw.	(11)

The term VH is called the Horizontal tail Volume ratio. The term η=QtQwis called the tail efficiency. Typically, 0.8<η<1.2. The variable εis called the tail downwash angle.

The Entire Plane (i.e. wing/body + tail):

For the airplane lift we have (for η=1):

CL=CLwb+StSCLt=CL0wb−StSCLαt[ε+(iw−it)] +CLαwb+CLαtStS α =Δ CL0+CLαα.	(12)

For the airplane moment we have:

Cm = (Cm0wb+Cmαwbα)+(Cm0t+Cmαtα)=Cm0wb+Cm0t+Cmαwb+Cmαt α =Δ Cm0+Cmαα.		(13a)

where, more explicitly:Cm0=Cm0wb+Cm0t = Cmacwb+CL0wb (h−hnwb)+ηVHCLαt[ε0+(iw−it)]		(13b)

and:                            Cmα = Cmαwb+Cmαt=CLαwb (h−hnw)−ηVHCLαt1−dεdα.		(13c)

[Note: Equations (13) are given in Nelson as equations (2.33-2.35). The latter equations include a fuselage term that is ignored in (13). Those equations also include the subscript ‘w’; whereas in (13) the subscript ‘wb’ is used.]

Example 1 [See Nelson EXAMPLE PROBLEMS 2.1 on p.49 and 2.2 on p.57.]

Given: the wing/body moment equation Cmwb=−.05−.0035 α, along with the following wing information:

S=178 ft2 ; b=35.9 ft ; c=5ft ; hwb=xcgc=0.1 ; AR=7.3 ;CLαwb=.07rad/deg ; CL(α=0)=0.26 ; iw=2o. Also, use the following information: lt=14.75ft ; ARt=4.85 ; CLαt=.073rad/deg ; CL(α=0)t=0.26 ;η=1.0.

Find: the tail Stand it so that the final plane has the moment equation: Cm=.15−.025 α

Solution:

(i)From (9) and (13b): Cm0t=Cm0−Cm0wb=.15−(−.05)=.20=ηVHCLαt[ε0+(iw−it)]=VH(.073)[ε0+(2o−it)] where from (11), ε0=2CLw(α=α0)π ARw=2(.26)π(7.3)=.0226rad=1.3o.

(ii)From (10) and (13c):  Cmαt=Cmα−Cmαwb=−.025−(−.0035)=−.0215=−ηVHCLαt1−dεdα|α0=−(1)VH(.073)1−dεdα|α0 where from (11): dεdα|α0=2CLαwπ ARw=2(.07)(180π)π(7.3)=0.35. Hence, we have .0215=(1)VH(.073)(.65), or VH=.453. From (11): St=VHcSlt=.453(5)(178)14.75=27.3ft2

Returning to (i): .20=.453(.073)[1.3+(2o−it)] gives it=−2.75o   □

The Stick Fixed Neutral Point- Recall from Definition 1 that the plane will have neutral stability when∂Cm(α)∂α|α=α0=Cmα=0. In view of (13c), this condition is:0=CLαwb (hn−hnwb)−ηVHCLαt1−dεdα. [Note that here we have defined the neutral point to be hn=Δh. Nelson denotes this as hNPin 
(2.36) on p.56. Note also that for the wing/body alone the neutral point is hnwb=hac In essence, it is how far the cg can be shifted back until the plane becomes neutrally stable.]

Hence, the plane neutral point is:         hn=hnwb+ ηVHCLαtCLαwb1−dεdα=hac+ ηVHCLαtCLαwb1−dεdα.  	(14)

Equation (14) is equation (2.36) on p.56 of Nelson.

Definition: The stick-fixed neutral point of a plane is defined as (14).

Notice that for the wing/body, hn=hnwb=hac. Whereas, for the entire plane it is greater by the amount given by the additional term in (14). From (11), we have dεdα|α0=2CLαwπ ARw. If this term is close to one, then the neutral point will be close to that of the wing/body. In other words, the tail contributes a ‘factor of safety’ in relation to how far aft the cg can move.  [Note: Equation (14) ignores the body contribution included in Nelson (2.36).]

Example 1 (continued). How much of a ‘cushion’ does the tail contribute to hn ?

Answer:  ηVHCLαtCLαwb1−dεdα=.0215.07=0.307  and h=xcgc=0.1. Hence, it contributes quite a bit! □

Clearly, the difference h−hnplays a major role in relation to longitudinal stability. For h<hn, we have Cmα<0, hence positive stiffness. This can always be achieved if the airplane CG is positioned sufficiently toward the front of the airplane. The difference h−hnis of such importance, that the negative of it is defined as the static margin: hn−h=ΔKn.

From (14), we have:                       hnwb=hn− ηVHCLαtCLαwb1−dεdα.	(15)

While (14) is convenient for some purposes, it is not convenient for clearly understanding how a change in the plane scaled  cg, h, influences its neutral point, hn. This is because the quantity VH=ltStcSincludes the term ltc=hnt−h Following Etkin, define ltc=hnt−hnwb. Then ltc=(ltc)−(h−hnwb). Hence,

VH=ltStcS=ltc−(h−hnwb)StS=ltStcS−StS(h−hnwb) =ΔVH−StS(h−hnwb).	(16)

At this point, rather than using the above final equations, it is more expedient to recall the following:

Mt=−Ltlt=−CLt12ρVt2Stlt ⇒ Cmt=Mt12ρV2Sc=−ηVHCLt.	(17)

Using (16), this becomes:            Cmt=−ηVH−StS(h−hnwb)CLt.	(18)

It follows that Cm=Cmwb+Cmt=Cmacwb+CLwb(h−hnwb)−ηVH−StS(h−hnwb)CLt.

Gathering terms:                  Cm=Cmacwb+CLwb+ηStSCLt(h−hnwb)−ηVHCLt.	(19)

At this point we will make the following: Assumption: η=Δ(VtV)2≃1. Then in view of the leftmost equality in (12), (19) becomes:     Cm=Cmacwb+CL(h−hnwb)−VHCLt.	(20)

From (20) we have:                 Cmα=CLα(h−hnwb)−VH∂CLt∂α.	(21)

From (21)  for the case of neutral pitch stability (Cmα=0)we have:0=CLα(hn−hnwb)−VH∂CLt∂α	(22)

From (22) we have:                           hn=hnwb+VHCLα∂CLt∂α	(23)

From (22) we also  have:                 hnwb=hn−VHCLα∂CLt∂α.	(24)

Substituting (24) into (21):                 Cmα=CLα(h−hn).	(25)

Equation (25) is exactly what one might expect, assuming that the neutral point is, indeed, the plane aerodynamic center.

For a given airplane configuration, suppose h is known. Solving (25) for hngives:

hn=h−(CmαCLα).	(26)

Now note that                            CmαCLα=Δ∂Cm∂α∂CL∂α=dCmdCL	(27)

equation (26) becomes:                      hn≃h−(dCmdCL).	(28)

Equation (28) provides an alternative to the use of pitch derivatives in experimentally estimating hn. Notice that it holds for any angle of attack, α. Experimental measurement of the amount of change in the pitch moment coefficient that results from a small change in the lift moment coefficient (generated by, for example, a small change in α) will yield an estimate of hn. The accuracy of such an estimate can be improved by repeating the test at a variety of specified values for α, and using the average.

Example 2. An aircraft is to be operated with its most rearward CG position limited to 25 ft. aft of the apex of the wing. The distance between the wing and tail mean aerodynamic centers is lt=55ft., the distance to the wing/body aerodynamic center is xacwb=21 ft., and the wing mean chord length is c=19.26ft. Estimate the area ratio StS, required to provide a control-fixed static margin, hn−h=Kn, of at least 0.05 at all times. Make the following assumptions:(A1) at=awb;(A2) hnw=hnwb;(A3) no power plant effects ;(A4) ∂ε∂α=0.25.

Solution: We are given:hc=25 ft, or . We are also given hnwbc=21 ft, or hnwb=2119.26 =1.09.

From (16): VH=StSltc−(h−hnwb) =StS5519.26−(1.30−1.09) =StS(2.646)

From (14):

Hence, hn=Kn+h =.05+1.30=1.35=1.09+1.985 StS. Hence, StS=0.131   □

2.3 Longitudinal Control

Deflection of the elevator through an angle δewill result in a change in both the lift and pitching moment of the aircraft. A downward elevator motion is taken to be positive. It will produce positive lift and negative pitching moment. If we assume these quantities are linear in δe, then we obtain:

CL=CLαα+CLδeδe	(29)

Cm=Cm0+Cmαα+Cmδeδe	(30)

Example 3. Consider the NAVION airplane described in Table B.1 on p.400 of Nelson.

(a)When flying in the equilibrium condition with δe=0, find the angle of attack, αo.

Solution: CL(α,0)=W0.5ρV2S=27500.5(.002377)(.158×1116.4)2(184)=.40417. Also, CLα=4.44rad.

Hence, αo=.404174.44=.091rad=5.214o

(b) Notice that Cm0is not given in Table B.1. Use your result in (a) to find it.

Solution: 0=Cm0+Cmααo ⇒ Cm0=−Cmααo=−(−.683)(.091)=.062rad.

(c) Find the values of α and δeneeded for a vertical acceleration of 0.1g’s.

Solution: Now the Lift is L=1.1 W=3025, and so CL(α,0)=.4041(1.1)=.4445. Hence, from (30):

0=Cm0+Cmαα+Cmδeδe or −Cm0=Cmαα+Cmδeδe. This, along with (29) give the matrix equation:

.4445−.062=CLαCLδeCmαCmδe αδe=4.44.355−.683−.923 αδe. Hence,  αδe=.1007rad.0073rad=5.77o−.42o.

(d) Is your answer your answer in (c) consistent with Figure 2.20 on p.63 of Nelson?

Answer: Yes. The trim angle of attack moved to the right when the elevator angle became negative. Specifically, for this elevator setting we now have

Cm(α,δe=−.42o)=Cm0+Cmαα+Cmδe(−.42×π180o)=Cm0+Cmαα+(−.923)(−.42×π180o)=(.0068+Cm0)+Cmαα=C(δe)+Cmαα

Notice that the Cm vs. α line has a smaller

intercept but has the same slope.

Q: What about the CL vs. αline?

The Derivatives CLδeand Cmδe(The following corresponds to the development in Nelson.)

The Lift Derivative CLδe: A deflection, Δδe, of the elevator will produce a lift force, ΔLt=ΔL, at the tail of the plane, and hence a change in the corresponding lift coefficient:

ΔCL=ΔL0.5ρV2S0.5ρVt2St0.5ρVt2St=VtV2StSΔCLt=ηStSΔCLt

Write ΔCLt≃dCLtdδe Δδe=dCLtdαt dαtdδe Δδe = CLαtτ Δδe where we have defined the parameter τ=dαtdδe. We then have: ΔCL=ηStSCLαtτ Δδe, or ΔCLΔδe=ηStSCLαtτ .

Hence:                                                     CLδe=η τStSCLαt .	(31)

The parameter τ=dαtdδeis shown as a function of SeSt

in Figure 2.21 on p.64 of Nelson.

Figure 2.21 on p.64 of Nelson

From Figure 2.9 of Nelson, the black line is the tail chord line for δe=0. The blue line is the tail chord line for a positive elevator angle (red).

Q: What about the CL vs. αline?

A: From (29): CL(α,δe)=CLαα+CLδeδe=CLαα+CL0(δe). In words, for a given δe, the lift line intercept is changed. The slope remains the same. This is shown

in the figure at the right.

The Moment Derivative Cmδe: The change in the plane pitching moment is described by:

Note that:ΔCm=−ηVHΔCLt=−ηVHCLαtτ Δδe, or ΔCmΔδe=−ηVHCLαtτ.

Hence:                                                     Cmδe=−η τVHCLαt.	(32)

The quantity in (32) is called the elevator control power. Using (16), we can also express (32) as:

Cmδe=−ηVH−StS(h−hnwb)CLαtτ.	(33)

Notice that (33) shows how the elevator moment derivative, Cmδe, changes, as a function of the plane scaled cg location, h.

Example 4. Consider again the NAVION plane addressed in Example 3. Find the elevator area, Se.

Solution:  While not given in Figure B.1 on p.401 of Nelson, in EXAMPLE PROBLEM 2.2 on p.57, we are given that St=43ft2. Hence, (see p.58), we have VH=0.66. Assume that η=1. From the top of p.58, we also have . Then from (32) and Table B.1 we have τ=−CmδeηVHCLαt=−(−.923).66(3.91)=0.358. For this value, our estimate of SeStfrom Figure 2.21 on p.64 of Nelson is: SeSt≃0.17. Hence, the elevator area is Se≃0.17St=.17(43)=7.31ft2.

Expressions for αtrimand δetrim-  In the case of a trimmed condition, where Cm=ΔCm(α,δe)=0, equations (29) and (30) yield:

αtrim=(CmδeCLtrim+CLδeCm0)/det 	(34a)

and                                        δetrim=−(CmαCLtrim+CLαCm0)/det	(34b)

where                                               det=CLαCmδe−CLδeCmα.	(34c)

SEE IF YOU CAN DERIVE (34) from (28) & (29).

NOTE: The three variables in (34) include CLtrim, αtrim, and δetrim. Specifying any one will determine the other two. For example, a specified value for CLtrimwill determine both αtrim and δetrim.

Solving (34a) for CLtrimgives:

CLtrim=CLα1−CLδeCLα CmαCmδe αtrim − CLδeCm0Cmδe.	(35)

Equation (35) is called the trimmed lift curve. There are a number of points to note in relation to this curve:

Point 1: The trimmed lift is a linear function of the trimmed angle of attack.

Point 2: The rate change in the trimmed lift is smaller for δe=0 [i.e. the basic condition shown in Figure 2.17 on p.37 of Etkin] than it is for δe=δetrim>0. Specifically, it is smaller by a fractional amount

0≤CLδeCLα CmαCmδe <1.

Note that since both Cmδeand Cmαare negative, this quantity is positive. (See Remark 1 on p.21 of these notes.)

Point 3: The zero-trimmed-lift angle of attack is no longer zero, but rather, is positive. Specifically, it is

αtrim= −CLδeCm0|Cmδe|1−CLδeCLα CmαCmδe ; for CLtrim=0

Point 4: The lift at αtrim=0is positive. Specifically:

CLtrim( αtrim=0)= CLδeCm0|Cmδe|.

These points are illustrated in Figure 2:

Figure 2. Plots of aircraft α(re: zero-lift) versus lift coefficient, CLfor basic and trimmed conditions.

Example 5. [Nelson. 2.4 on p.86] The Cmversus αcurves for various values of δeare shown below. We also have: (i) CL=.03+.08 α (deg.), and (ii) −15o≤δe≤15o.

(a)Estimate the fixed stick neutral point.

Solution: From (26), we have hn=h−(CmαCLα). From the above figure, Cmα=dCmdα−.3−(−.12)15=−.012/deg. From (i) we have CLα=.08/deg. From the upper right corner of the figure, we have h=.25. Hence,

hn=.25−(−.012.08)=.4.

(b)Estimate the control power, Cmδe.   Solution: Cmδe=dCmdδe−.12−.0210−5=−.028/deg

(c)Find the forward cg limit.

Solution:

- Clearly, the largest value of h is the neutral point, hNP=hn. A larger value will result in a plane that is statically unstable. The smallest value of h is not related to stability, but rather to the ability to trim the plane at large values of CL(e.g. when landing the plane). From the figure, we have:

CLmax=.03+.08 (15o)=1.23.

- As the cg is moved forward, the  Cmversus αcurves in the figure will become steeper. In other words, the magnitude ofCmαwill increase. This must be countered by an increase in the elevator moment. If we assume that Cmδeis not significantly influenced by the movement of the cg, then the maximum positive elevator moment is:

Cmmax=ΔCmδeδemax==−.028 (−15)=0.42.

- From (30) at the trim condition, we have: Cmtrim=0=Cm0+Cmααmax+Cmδeδemax, or

Cmα=−(Cm0+Cmδeδemax)αmax.

From the figure (δe=0oline in RED), m0≃0.17. Hence, Cmα=−(.17+.42)15 = −.039.

- Again, from (26) we have: h=hn+(CmαCLα)=0.4+(−.039.08)=−.087. In words, the forward cg limit is slightly ahead of the front of c.    □

Example 6. [Nelson Problem 2.10 on p.90] The airplane in Example 5 has the following hinge moment characteristics:

CLαw=.09/o ; CLαt=.08/o ; Chα=−.003/o ; Ch0=0 ; Chδ=−.005/o ; Ch0=0 ; SeSt=.35 ; VH=.4 ; dεdα=.4.

Find the stick free neutral point.

Solution: From (2.64) on p.70: hn'=hn−(1−f)VHηCLαtCLαw1−dεdα where f=1−CLδeChδeChαtCLαt.  The only quantity not given is CLδe. From (31), we have CLδe=η τStSCLαt . The only quantity in this expression not given is τ; which can be estimated from Nelson FIGURE 2.21 on p.64 as: τ≃.55. We then obtain CLδe=.0154/o, and then f=.88. From Example 5 we found hn=.4. Finally, hn'=.4−.0256=.3744. Notice that this stick free neutral point  lies ahead of the stick fixed neutral point.   □

2. Yaw (Directional) Static Stability [Nelson p.73]

The yaw angle, β, is defined to be positive if the

plane is yawing left, as shown in Figure 2.28 of Nelson.

Recall that clockwise moments are defined to be positive

moments. Hence, for the situation shown in this figure,

a positive tail rudder moment is needed to eliminate

the yaw. The graph in this figure reflects this fact, in

relation to the yaw moment derivative, Cnβ.

Definition 2.2.1 An airplane posseses directional,

or weathercock stability if Cnβ>0.

The yaw moment has two contributions: one associated

with the wing/fuselage, and the other associated with the

rudder.

Figure 2.28 on p.73 of Nelson. Static directional stability.

The wing/fuselage contribution-

Repeating (2.73) on p.74 of Nelson here:                 Cnβwb=−knkRlSfslfSb (/deg).                      (36)

The quantities S and b are the wing area and length, respectively. The new quantities introduced in (36)

include:

kn= an empirical wing/body interference factor that is a function of the fuselage geometry.

kRl= an empirical correction factor that is a function of the fuselage Reynolds number.

Sfs= the projected side area of the fuselage.

lf= the total length of the fuselage (from nose tip to back of tail).

The parameter kRlis related to the body Reynolds number

Rlf=Vlfν as shown in Nelson Figure 2.30 at the right.

[Note: νis the kinematic viscosity of the air.]

Figure 2.30 (p.75) Reynold number correction.

While kRlis straightforward to obtain. The interference factor knis not. For this reason, I have chosen to include Figure 2.29 (p.75), along with enhancements, on the following page of these notes.

Figure 2.29 on p.75 of Nelson. Wing/body interference factor kn

The path in red illustrates how kn is arrived at. One begins with knowledge of xmlf, where xmis the distance from the tip of the nose to the plane cg. Knowledge of (lfSfs)2gives the first leg of the path. Knowledge of the shown fuselage height ratio h1h2gives the second leg. Knowledge of the ratio hwf

completes the third leg, allowing one to then estimate knin the fourth and final leg.

The vertical tail contribution-

The magnitude of the side force acting on the vertical tail can be expressed as:

Yv=−CLαv αv(QvSv)   where we have assumed CLαv>0.  (2.75)	(37)

The vertical angle of attack, αv is :          αv=β+σ	(38)

where σis the sidewash angle [See Figure 2.31 on p.76 of Nelson]. From (37) and (38) we have:

CLv=−CLαvαv=−CLαv(β+σ)     .	(39)

From (39):                            dCLvdβ = −CLαv1+∂σ∂β =Δ CLβ. 	(40)

It is important to note that CLβ<0 is necessary for yaw stability. It is also important to have the explicit definition (40) of  CLβ. No such definition is given in the development on pp.74-77 of Nelson; even though this parameter is one of the many included, for example, in the NAVION Table B.1 on p.400.

It follows that the restoring moment produced by the vertical tail is:

Nv=−Yvlv=lvCLαv(β+σ)(QvSv)=−lvCLβ1+∂σ∂β(β+σ)(QvSv).	(41)

This moment is positive due to the fact that the force (37) is in the negative y direction, and it is applied at the negative x location lv.

Note: Equation (37) is consistent with (2.74) on p.74 of Nelson. However, the middle equality in (39) is not consistent with the middle equality in (2.76). This differs from (39) by a minus sign. The rightmost equalities of (39) and (2.76) are the same. Hence, it would appear that a minus sign is missing from the middle equality of (2.76). While this point may seem minor, it can cause confusion as to the sign convention concerning the yaw angle, β, and the sidewash angle, σ. Specifically, (38) suggests that σincreases the rudder angle of attack, αv. Now, consider the figure below.

[From Mechanics of Flight by W. Phillips]

This figure illustrates that the sidewash actually decreases the rudder angle of attack. In other words, (38) should be αv=β−σ. In Etkin we find αv=−β+σ; but only because of his sign convention for αv. In fact, as noted by Phillips, there is no accepted convention for either the yaw or the sidewash angle. The only way that Nelson’s expression (38) is consistent with sidewash reducing the rudder angle of attack is if σis assumed to be negative. If we make this assumption, then the yaw moment derivative given in (43) below is actually reduced as a result of the sidewash gradient.

From (41) we have:

Cnv=−VvηvCLβ1+∂σ∂β(β+σ)	(42)

The vertical tail stability derivative is, therefore:

Cnβ=Δ∂Cn∂β = −VvηvCLβ.	(43)

Clearly, from (40) and (43) we have CLβ<0 and Cnβ>0. Even so, in the case of the NAVION plane in Table B.1 on p.400 of Nelson, we see that CLβ=−.074 and Cnβ=−.071. No, we are given Clβ=−.074. Even so, we know that we still have CLβ<0. We can deduce that there is a typo in this second term, and that it should be Cnβ=.071. Were this not the case, then the NAVION plane would not have weathercock stability!

Nelson then offers on p.76 the following “simple algebraic equation”:

ηv1+dσdβ=0.724+3.06SvS1+cosΛc4w+0.4zwd+0.009ARw	(44)

where zwis the distance. parallel to the z-axis, from the wing root quarter chord point to the fuselage centerline, where d is the fuselage maximum vertical depth, and where Λc4wis the sweep of the wing quarter chord (i.e. the amount of angle that the wing is ‘swept’ back from perpendicular to the body at the ¼ chord location).

Yaw Control Using the Rudder-

We must be clear as to the sign convention related to the rudder.

For this reason, we include Figure 2.32 on p.77 at the right.

From this figure, we see that if the yaw angle βis positive, then

the rudder angle δrshould be negative in order to right the plane.

Applying a negative δrwill produce a negativeCLr=CLδrδr. This

implies that the rudder lift derivative CLδr>0. Hence, the rudder

moment will be:	Figure 2.32 of Nelson.

Cnr=−VvηvCLr = −VvηvCLδrδr =Δ Cnδrδr.	(45)

The rudder moment derivative Cnδr=−VvηvCLδr is referred to as the rudder control effectiveness.

The rudder lift derivative given in (2.86) on p.78 is:

CLδr=ΔdCLvdδr=dCLvdαv dαvdδr=ΔCLαvτ	(46)

where the parameter dαvdδr=Δτcan be estimated from Figure 2.21 on p.64. Using (40):

dCLvdβ = −CLαv1+∂σ∂β =Δ CLβ	(40)

we can also write (46) as:

.

REMARK 1: From (43) and the leftmost equality in (45) we have:      CnβCLβ=CnδrCLδr. 	(47)

To prove that this result leads to the definition of τ=Δ dαvdδrin (46). Suppose that (47) condition holds, and suppose that (46) holds:CLδr=CLαvτ. We will prove that τ= dαvdδr.

Proof: CnβCLβ=CnδrCLδr=CnδrCLαvτ ⇒ τ=CnδrCnβ CLβCLαv=dCndδrdCndβ dCLdβdCLdαv=dβdδr dαvdβ=dαvdδr □

From the above proof, it follows that:                     τ=CnδrCnβ CLβCLαv.	(48)

From (40), we have:                             CLαv = −CLβ1+∂σ∂β.	(49)

Substituting (48) into (49) gives:                         τ = −CnδrCnβ 1+∂σ∂β.	(50)

If the sidewash derivative ∂σ∂βis known, then τcan be computed directly, as opposed to being estimated from Figure 2.21 on p.64. Notice that because Cnδr<0 and Cnβ>0, it follows that τ>0.

From (50) we also have:                              ∂σ∂β = −1+τ Cnβ−Cnδr .	(51)

Example 7. Find the permissible range of values for ∂σ∂βfor the NAVION plane.

Solution: The derivatives given in Table B.1 on p.400 of Nelson are:

Clβ=−.074rad ; Cnβ=−.071rad ; Clδr=.107rad ; Cnδr=−.072rad.

Before proceeding any further, two points related to these values must be made:

Point 1: Since Cnβ<0corresponds to weathercock instability, there is a mistake. We will assume it is a simple sign error. Hence, Cnβ=.071.

Point 2: The parameters Clβ=−.074rad ; Clδr=.107rad  are not the parameters CLβ ; CLδr. They are roll moment derivatives associated with the side lift forces Lβand Lδr. As such, they are related to the vertical tail cg. This point illustrates the importance of notation. For example, on p.21 we have (1.64):

L=ClQSl      Rolling moment.

Hence, Clis a scaled moment. It then follows that, for example, Clβ=dCldβ=Clβ.

From (51) we have: ∂σ∂β = −1+τ Cnβ−Cnδr =−1+.071 τ.107=−1+.66 τ.

From Figure 2.21 on p.64 we observe that 0≤τ≤0.8. Hence, −1≤∂σ∂β≤−0.47. Notice that the sidewash gradient must be negative. This is consistent with the discussion following (42) above. □

Example 8. [Problem 2.13 on p.91 of Nelson]  Size the vertical tail for the airplane configuration shown below so that its weathercock stability parameter,Cnβ=+0.1rad. Assume V=150msat sea level.

S=21.3m2 ; b=10.4m ; zw=0.4m ; d=1.6m ; lf=13.7m ; xm=8m ; wf=1.6m ; Sfs=15.4m ; h=h1=1.6m; h2=1.07m

Solution: From (36) we have: Cnβwb=−knkRlSfslfSb (/deg). We desire to have Cnβ=0.1rad=.0017/o.

From xmlf=813.7=0.58and Figure 2.29 (red), we have kn≃.0015. At sea level, ν=1.46(10−5)m2s, and so Rlf=141(106). In relation to Figure 2.30, this gives kRl≃2. Hence,

Cnβwb=−.0015(2)(15.4)(13.7)21.3(10.4)=−.0029/o=−.166rad.

Hence, the tail must contribute Cnβt=Cnβ−Cnβwb=0.1−(−.166)=.266rad. And so, from (42):

Cnβt=.266=VvηvCLαv1+dσdβ.

Now: Vv=lvSvSb=lvSv21.3(10.4)=.0045 lvSv. Also, from the ‘simple algebraic equation’ (44):

ηv1+dσdβ=.274+3.06 SvS1+cosΛc4w+.4 zwd+.009 ARw,

where ARw=b2S=10.4221.3=5.08, and where the maximum fuselage depth d=h1=1.6. Hence,

Cnβt=.266=(.0045 lvSv)CLαv.274+3.06 Sv21.31+cosΛc4w+.4 .41.6+.009 (5.08), or

.266=(.0045 lvSv)CLαv.274+.1437 Sv1+cosΛc4w+.1457.	(e7.1)

There are a number of parameters related to (e7.1) and (e7.2) that still are not known. In the solution manual of Nelson, it is now assumed that: lv=4mand CLαv=0.1/o=5.73rad. The solution makes no mention of an assumption of a value for Λc4w

Here, we will asume Λc4w=15o=.262rad.

Then (e7.1) becomes:

.266=.103Sv.073Sv+.42 ⇒ .073Sv2+.42Sv−2.58=0 ⇒ Sv1,2=−9.48 , 3.73. Hence, Sv=3.73 m2

[The author’s answer was Sv=3.9m2; the difference due to his choice of a different value for Λc4w.] □

QUESTION: For the designed vertical tail, does the sidewash gradient have a stabilizing or destabilizing effect? In other words, does it make Cnβlarger or smaller?

ANSWER: ηv1+dσdβ=.274+3.06 3.921.3S1+cos(.262)+.4 .44+.009 (5.08)=.645. Hence, dσdβ=.645ηv−1. In words, the sidewash gradient will b negative for ηv=VvVw2>.645, or for VvVw>0.8. It is hard to imagine that a speed ratio less than 0.8 is possible. Hence, we can conclude that the sidewash gradient is negative. Because it is negative, it reduces the yaw derivative, and so has a destabilizing effect per this metric. [Note: From (39): CLv=−CLαvαv=CLαv(β+σ)and the fact that σ=(∂σ∂β) β with ∂σ∂β<0, we see that the amount of yaw is reduced when the sidewash angle is included. While this is a good thing, our definition of static stability is related to the moment needed to right the plane; not to the amount of yaw the plane is experiencing.]

2.3 Roll Stability [p.78 of Nelson]

On p.78 we have the following definition of static roll stability:

“An airplane posseses static roll stability if a restoring moment is developed when it is disturbed from a wings-level attitude.”

REMARK 3. The author then goes on to say: “The roll moment created on an airplane when it starts to sideslip depends on the wing dihedral, …”, and later, “When an airplane is disturbed from a wings-level attitude, it will begin to sideslip…”. These two statements suggest that, on the one hand, a small roll angle will generate sideslip that, in turn will generate more roll, while on the other hand, it is sideslip that generates the roll. If, for example, a lateral wind gust were to generate sideslip with the wing-level attitude maintained, would the plane begin to roll? I would suspect not. Now, one might argue that, in reality, there would always be at least a slight amount of roll generated by such a gust. And, in the case of a positive dihedral, this would be a positive roll. I can accept this explanation. However, I would then argue that it is the roll perturbation that is the origin of the roll, and that the attendant sideslip simply increases the roll once it has begun.

Nelson illustrates the development of roll in Figure 2.33 on

p.79 (shown at the right here). In this illustration only roll

is indicated. It may well be that the roll perturbation will

result in attendant sideslip. [I am by no means an expert in this

area, and so I would accept such a possibility.] [Notice that

in this figure the term C1β is a typo. It should be Clβ. This

figure is from Nelson’s first edition. It is corrected in the

second edition.]

Figure 2.33 on p.79.

Nelson then goes on to state, as well as illustrate in this figure, that “The requirement for (roll) stability is that Clβ<0.” Unfortunately, he never gives an explicit expression for Clβ. [As stated in Point 2 on p.18 of these notes, Cnβis the yaw moment (about the vertical tail cg) derivative. ] From p.21 we have L=roll moment. Hence, Cl =L/(QSl). Hence, Clβ= dCl/dβ. Nelson presumes the student is well aware of this notation. With this in mind, it should then be clear that for a perturbed positive roll, L, caused by side slip β, the restoring moment must be negative. In this event, we will have Clβ<0.

The geometry associated with a roll angle φis shown in Figure 2.34 on p.80:

Figure 2.34 Influence of the dihedral Γon the lateral velocity of each side of the wing for a roll angle φ.

The roll angle φ differs from the angle of attack, α, and the sideslip angle, β, in that its reference is not the plane body coordinates, but rather the direction of gravity. In order to gain a better picture of exactly how φrelates to the roll moment derivative Clβ, we will briefly digress from Nelson (where it is not clear at this point) to Etkin. However, we will begin this digression by recalling Figure 1.10 on p.20:

This is L

In the absence of any sideslip (β=0) and of any roll (φ=0), the velocity relative to the plane body coordinates is:

V=Vcosαx0sinαx=uvw.	(E3.11.1)

After rolling through an angle φ>0 about the body x-axis, it can be shown that the velocity becomes:

V'=V10000sinφ00cosφcosαx0sinαx=Vcosαxsinαxsinφsinαxcosφ=uv'w'.	(E3.11.2)

Notice that the roll has not only reduced the velocity component w. It has also created a sideslip velocity v'=Vsinαxsinφ! From equation (1.68) of Nelson, we have:

β=sin−1v'V=sin−1(sinαxsinφ)≃αxsinφ≃αxφ.	(E3.11.3)

The second rightmost approximate equality assumes small αx. If, in addition, we assume that φis also small, then we obtain the rightmost approximate equality. Equation (E3.11.3) is exactly the relation between φand β that Nelson omits.

As a result of this roll-induced positive sideslip,β, the plane will have roll static stability (i.e. positive roll stiffness) if the plane generates a negative roll moment. This roll moment is denoted as:

Cl=Clββ=Clβαxφ.	(E3.11.4a)

The roll moment derivative is:                          Clφ=Clβαx.	(E3.11.5b)

Hence, for static roll stability we must have Clβ<0AND αx>0.

Remark. The quantity Clβappears to be the roll moment derivative with respect to the sideslip angle, β. Indeed it is. However, the sideslip angle was generated by the roll angle φ. The chosen notation for Clβhides this fact.

For small sideslip this becomes: β=vu. Because of the dihedral, the change in the angle of attack of the right (lower) wing is:

Δα=vnu=vsinΓu≃vΓu=β Γ	(56)

The change in the left (upper) wing angle of attack is simply the negative of (56).

Roll Control-

Roll control is achieved by differential deflection of the ailerons. In relation

to Figure 2.36. consider an incremental lift, Δ(y),at a location y along the

aileron. The corresponding rolling moment increment is:

ΔL=Δ(y)×y = [CℓQ(c dy)]×y.                       (57)

In (57) the parameter Cℓis the aileron section lift coefficient. For clarity,

we will rename this as CLa. Note that this is not the same ‘animal’

as CLα, which is a pitch lift derivative. Nor is it the roll moment coefficient      Figure 2.36 on p.82

Cl !!!  Scaling (57) by

QSb gives:                 ΔCl=ΔLQSb = CLacy dySb.                                 (58)                     We will now express the aileron lift coefficient CLaas:        CLa=CLαaαa.	(59)

From (59) we then have:               CLa=CLαaαa=CLαaαaδa δa =Δ CLαaτ δa.	(60)

ΔCl= (CLαaτ δα)cy dySb	(61)

If we multiply (61 ) by 2 in order to account for both ailerons, and integrate over the aileron span, we obtain:

Cl= 2CLαaτ δαSby1y2cy dy.	(62)

Hence:                                                  Clδa= 2CLαaτ Sby1y2cy dy.	(63)

Notice in (63) Nelson uses the notation Clαwinstead of Clαa, suggesting that this lift coefficient derivative is equal to the wing lift coefficient derivative. In view of this, the following explicit assumption should be stated: “We will assume that Clαa=Clαw”.

The Influence of the aileron width and position on the control power-

Recall that for a tapered wing:                    c=cr1+λ−1b2 y.	(64)

From (64) we see that the wing tip chord length is a fraction, λ, of the the root chord length.

Hence, y1y2cy dy= cry1y21+λ−1b2 yy dy = cry22+λ−1b2 y33y1y2=cr2y22−y12+43λ−1b y23−y13.	(65)

Now, it is easy to show that: y22−y12=2Δaμa   and    y23−y13=Δa(3μa2+.25 Δa2)	(66)

where Δa=y2−y1is the aileron width, and  μa=y1+y22is the position of the center of the aileron along the wing. Substituting (66) into (65) gives:

y1y2cy dy=crΔaμa−1−λb (2μa2+.167 Δa2)                                      .	(67)

Substituting (67) into (63) gives:

Clδa= 2CLαaτ crΔa Sbμa−1−λb (2μa2+.167 Δa2).	(68)

Special Case: Suppose that2μa2+.167 Δa2=2(μa2+.083 Δa2) ≃2μa2. Then (68) becomes:

Clδa≃ 2CLαaτ crΔaμa Sb1−1−λb2 μa.	(69)

The moment derivative (68) is called the aileron roll control power. Equations (68) and its approximation, (69), give an explicit description of the influence of various key parameters on the control power. For example, as the term 1−λb 2μa=(1−λ)μab2→1 the control power goes to zero. Let μa(b2)=Δra denote the ratio of the aileron center position to the half-wing span. Then 1−λb 2μa=(1−λ) ra. For this quantity to approach 1, we must have λ→0 and ra→1. The first condition is physically possible, but the second is not for an ailerion with width Δ>0. Even so, we see clearly how λ and ra influence the relative static roll stability.

The optimum aileron position-

∂Clδa∂μa=0=1−1−λb 4μa ⇒ μaopt=b22(1−λ). For λ=0.5, μaopt=b2, which implies that the center should be as near to the tip as possible. Forλ=0.25, μaopt=b3. □

Example 9 [Problem 2.14 on p.91 of Nelson]

Figure P2.14 shows the wing planform for an aivation business

airplane. Determine the roll control power. [Assume that the

aileron section lift coefficient is CLαa∞=0.1/o.

Solution:

Step 1:  S=21.3m2 and AR=b2S=5.08. Hence,

CLαa=CLαa∞1+CLαw∞(π AR)=5.751+5.73(π∗5.08)=4.21rad

Step 2: Using cac=.25in Figure 2.20 gives τ=0.48

Step 3: λ=1.342.75=.487. Δa=4.8−3.4=1.4. μa=4.8+3.42=4.4.

Step 4: Clδa= 2CLαaτ crΔa Sbμa−1−λb (2μa2+.167 Δa2)=0.17rad.   □