General Equations of Unsteady Motion

[The following notes are related to Chapter 3 of Nelson.]

3.2 Derivation of the Rigid Body Equations of Motion

In these note we will not repeat the development of these equations. Instead, we will simply clearly define the many variables that comprise them. In the plane-body coordinate system we have:

r=(x,y,z)=position  ;  V=(u,v,w)=linear velocity  ; ω=(p,q,r)=angular velocity (roll,pitch,yaw)

A=(X,Y,Z)=  force      ; G=(L,M,N)=moment ;

h'=(hx,hy,hz)= angular momentum of spinning rotors re: rotor center of mass.

The development in section 3.2 yields the final important moment equations:

L = Ixp−Izxr+qr(Iz−Iy)−Izxpq+qhz'−rhy'	(3.26a)

M= Iyq+Izxr+rp(Ix−Iz)−Izx(p2−r2)+rhx'−phz'	(3.26b)

N = Izr−Izxp+pq(Iy−Ix)+Izxqr+phy'−qhx'	(3.36c)

3.3 Variables that Relate the Aircraft Body to the Earth (the Euler Angles)

The above equations of motion were derived for a moving coordinate system fixed to the plane.

The orientation of FB relative to FE is given by a series of three ordered rotations:

ψ= yaw rotation of (xb,yb,zb) about zb  to get (xE,y1,zb) [azimuth]

θ= pitch rotation of (xE,y1,zb) about y1  to get (xE,yE,z2) [elevation]

φ= roll rotation about of (xE,yE,z2) about xE to get (xE,yE,zE) [bank angle]

The following is a picture of the first rotation:

xEy1=CψSψ−SψCψ xbyb or xEy1z1=CψSψ0−SψCψ0001 xbybzb=ΔRzbxbybzb   (1a)

The matrix Rzb=Rzb(ψ)is the rotation matrix about the zbaxis.

[Note: There is a sign error in the leftmost equation of (3.27) on p.102.]

The second rotation

will be about the y1axis: xEyEz1=Ry1(θ) xEy1zb, 	(1b)

and the third will be about the xEaxis: xEyEzE=RxE(φ) xEyEz1.	(1c).

Equations (1) result in:

xEyEzE=RxERy1Rzbxbybzb=Rxbybzb=CθCψ(SφSθCψ−CφSψ)(CφSθCψ+SφSψ)CθSψ(SφSθSψ+CφCψ)(CφSθSψ−SφCψ)SθSφCθSφCθ xbybzb.	(2)

Taking the time derivative of (2) gives:

xEyEzE=Ruvw=CθCψ(SφSθCψ−CφSψ)(CφSθCψ+SφSψ)CθSψ(SφSθSψ+CφCψ)(CφSθSψ−SφCψ)SθSφCθSφCθ uvw.	(3.30)

Equations (2) and (3.30) relate the plane body position and velocity to its position and velocity referenced to the earth coordinates, when one is given the Euler angles , which are the roll, pitch, and yaw angles.

In the same manner that (2) was arrived at, using Figure 3.3 on p.102 we can arrive at:

pqr=10−Sθ0CφCθSφ0−SφCθCφφθψ =Δ Qφθψ	(3.31)

Hence,                            φθψ=1SφtanθCφtanθ0Cφ−SφSθSφsecθCφsecθ pqr=Q−1pqr.	(3.32)

Equation (3.31) is a first order nonlinear differential equation that can be numerically integrated to obtain at a time t, when one has (p.q.r)for the time interval [t0,t].

3.4 Gravitational and Thrust Forces and Related Moments

This material is given on pp. 103-104 of Nelson.

Fxgr=−mgsinθ ; Fygr=mgcosθsinφ ; Fzgr=mgcosθcosφ	(3.33)

The propulsion system forces and moments are denoted using the subscript ‘p’ in Table 3.1 below.

Table 3.1 Summary of kinematic and dynamic equations

X−mgsinθ = m(u+qw−rv)

Y+mgcosθsinφ = m(v+ru−pw)	force equations

Z+mgcosθcosφ = m(w+pv−qu)

L= Ixp−Ixzr+qr(Iz−Ix)−Ixzpq

M= Iyq+Ixzr+rp(Ix−Iz)−Ixz(p2−r2)	moment equations

N = Izr−Ixzp+pq(Iy−Ix)+Ixzqr

pqr=10−Sθ0CφCθSφ0−SφCθCφφθψ =Δ Qφθψ	body angular velocities

φθψ=1SφtanθCφtanθ0Cφ−SφSθSφsecθCφsecθ pqr=Q−1pqr	Euler rates

xEyEzE=Ruvw=CθCψ(SφSθCψ−CφSψ)(CφSθCψ+SφSψ)CθSψ(SφSθSψ+CφCψ)(CφSθSψ−SφCψ)SθSφCθSφCθ uvw	fixed frame velocities

3.5 Small Disturbance Theory

Notation: The reference value of a variable, say, γ, is denoted as γ0. The small disturbance of γabout γ0 is denoted as Δγ. If γ0=0, then the small disturbance is simply denoted as γ.

Definition 1. The reference flight condition (relative to the stability axes) for an aircraft is defined as symmetric flight with no angular velocity. Mathematically, this condition is:

ω0=[p0,q0,r0]tr=0  ;       V0=[u0,v0,w0]tr=[u0,0,0]tr      ;       (φ0,θ0,ψ0)=(0,θ0,0).

The X-Force Equation-

From Table 3.1 we have:                 X=mgsinθ + m(u+qw−rv).	(3.38)

dX=∂mgsinθ∂θ|θ=θ0dθ + m∂u∂u|u=u0du + ∂(qw)∂q|q,w=0dq + ∂(qw)∂w|q,w=0dw− ∂(rv)∂r|r,v=0dr − ∂(rv)∂v|r,v=0dv.

Hence,                                               dX = mgcosθ0dθ + du.	(3.44)

Equation (3.44) show that a small change in the X force is related to a small change elevation, dθ, and to a small change in the body forward velocity du.

Next, assume:                       dX = ∂X∂udu + ∂X∂wdw + ∂X∂δedδe + ∂X∂δTdδT	(3.45)

or,                                         dX = Xudu + Xwdw + Xδedδe + XδTdδT.	(3.45’)

Equating (3.44) and (3.45) gives: ddt−Xudu − Xwdw + mgcosθ0dθ = Xδedδe + XδTdδT.	(3.47)

The following table includes the linearized rigid body equations of motion.

3.6 The Force and Moment Equations

The equations in Table 3.2 above include the force and moment derivatives. In this section, we look at these derivatives in greater detail. Prior to doing that, it is expedient to scale those derivatives, so that they are dimensionless. Recall that, given a force, X, we can scale it by QS, which also has units of force, so that Cx=XQSis dimensionless. Now consider the force derivative ∂X∂u, the units of which are force divided by velocity. The natural scale factor for this is u0QS:

CXu=Xuu0QS.	(3.51)

From (3.51) we have:      CXu=Xuu0QS=∂X∂uu0QS=∂(XQS)∂uu0=∂CX∂uu0=∂CX∂(uu0).	(3.52)

3.6.1 Derivatives Due to Change in Forward Speed

Change in X-

The main forces that influence forward speed are drag and thrust. Write:

Nelson then claims that (3.55) can be written as:

Xu = −QS∂CD∂u+2CD0u0 + ∂T∂u.	(3.56)

Remark 1.  I personally do not see how this follows. You might recall that the drag force coefficient is: CD=CD0+KCL2. If we take the derivative of this with respect to u, the constant CD0vanishes. I will have to pursue this further, and get back to you on it.

10/14: Well, I’m back :) Define CD0=CDmin+KCL02, associated with angle of attack α0. Write

D(u)=D0(u)+∂D∂u|u0du=CD012ρV2S +CDuu0du12ρu02S.    (*)

Next, note that:         ∂CD012ρV2S ∂u|u0=CD0ρ u0=2CD0u012ρV2S. It follows that the derivative of (*) is:

∂D(u)∂u=QSu02CD0+CDu . We can conclude that the author is using CD0=CDmin+KCL02; not CD0=CDminas I had presumed. This is important because the author’s definition entails a presumed α0. One would reasonably expect this to be the equilibrium (steady level flight) condition. However, it need not be a level flight equilibrium.

Scaling (3.56) as was done in (3.51) gives:

CXu = −∂CD∂(uu0)+2CD0 + CTu = −CDu+2CD0 + CTu.	(3.57)

The term CDucan be estimated from a plot of CD versus Mach number, M, as:

CDu≃M ∂CD∂M|M=M0.	(3.59)

Remark 2. Nelson notes that the approximation CTu≃0is valid for gliding flight and for jet-powered aircraft. For variable propellor pitch and piston engine aircraft it is approximated as CTu≃−CD0. This would be the case for the NAVION: In this case, CDu≃0. Hence, CTu≃−CD0 But if this is true, then the thrust influence is in the SAME direction as the drag. The coefficient CD0 is POSITIVE. Its direction is accounted for by the negative sign in front of it. For the NAVION plane we have CD0=.05. If we accept Nelson’s claim then CXu=−3CD0.

QUESTION: Why is Xu so important NOW- when we are completing the study of the phugoid mode in Chapter 4?

Change in Z and M-

In a manner similar to (3.57) we have:

CZu = −CLu+2CL0	(3.61)

where CLucan be approximated as:         CLu≃M21−M2CL0.	(3.66)

The pitch moment due to a small change in forward speed is given as:

∂M∂u = CmuQS {cu0=Cmuρu0Sc.	(3.69)

Remark 3. It appears to me that (3.69) is missing a factor of 0.5.