CHAPTER 4    Longitudinal Motion

Read section 4.2.

Longitudinal Equations

ΔuΔwΔqΔθ=XuXw0−gZuZwu00Mu+MwZuMw+MwZwMwu0+Mq00010 ΔuΔwΔqΔθ +XδZδMδ+MwZδ0 XδTZδTMδT+MwZδT0ΔδΔδT (4.51)

PURE PITCHING MOTION (Section 4.3 on p.139 of Nelson)

EXAMPLE PROBLEM 1 It is desired to develop a test rig for estimating the pitching lift derivative coefficient, CLα, for horizontal tail designs. The beginning of this development is shown below.

Figure 4.9 of Nelson Rod-plate assembly constrained to pure pitching motion.

(a) Development of the equation of angular motion:

We begin with:            Mcg=Iyθ = ∂M∂α0α + ∂M∂q0q .	(E1.1)

Note #1: Since the cg of the test rig is constrained, the angle of attack, α, and the pitch angle, θ, are one and the same. Hence, α=θ and q=θ.

Note #2: Let Mα=Δ ∂M∂α0×1Iy     and     Mq=Δ ∂M∂q0×1Iy.

From these notes, (E1.1) can be written as:

θ−Mqθ−Mαθ=0.	(E1.2)

(b) Development of the relation between(Mα,Mq)and CLα:

M(α)= −l×(CLαα)×12ρu02S ⇒ Mα=−CLα12ρu02SlIy.	(E1.3a)

Recall that tanα≃qlu0, so that for small α, we have α≃qlu0. Hence,

M(q)= −l×CLα×qlu012ρu02S ⇒ Mq=−CLαlu012ρu02SIy.	(E1.3b)

From (E1.3a-b) we obtain:                           Mq=lu0 Mα.	(E1.3c)

Substituting (E1.3c) into (E1.2) gives:

θ+lu0Mαθ+Mαθ=0.	(E1.4)

(c) Relation between Mα and the transient response associated with (E1.4):

[Subtitled: How many different way can we estimate CLα?]

We know that the transient response associated with (4) will be decaying and oscillatory. Hence, we can write the left side of (E1.4) as:

θ+lu0Mαθ+Mαθ =Δ θ+2ςωnθ+ωn2θ.	(E1.5a)

From (E1.5a), we have:                            ωn=Mα.	(E1.5b)

We also have lu0Mα=lu0ωn2=2ςωn. This gives:       ς=ωnl2u0=Mα2u0.	(E1.5c)

Also, the system time constant is: τ=1ςωn=2u0Mα.

Now, the transient response associated with (E1.5a) has the form:

θ(t)=θ1 e−ςωnt sin(ωdt +ϕ1)	(E1.6a)

where the pair of constants (θ1,ϕ1)will depend on the type of specified initial conditions . The initial conditions are not as important as the nature of the response (E1.6a). If we define τ=Δ1ςωn, then (E1.6a) becomes:

θ(t)=θ1 e−tτ sin(ωdt +ϕ1).	(E1.6b)

The parameter τis called the time constant associated with (E1.5a). This gives rise to one method for estimating CLαfrom the transient response measurement.

Method 1.  Ignore the oscillations and use only the decay envelope θ1 e−tτ: At a time t=τ, this envelope will equal θ1 e−1≃0.37θ1. And so, to estimate t=τ, we sketch an exponential envelope on the decaying oscillatory response, and find the time at which this envelope is ~37% of its peak value. Having this estimate, call it τ, we then note that

1τ=l2u0Mα=CLαρu0Sl24Iy.

And so, our estimate of the magnitude of CLα, call it CLαis:      CLα=ρu0Sl24Iy−11τ.

Method 2. Ignore the envelope and use only the oscillation frequency: The oscillation frequency, ωd, of the decaying response (E1.6) is: ωd=ωn1−ς2. From (E1.5b) and (E1.5c) this becomes:

ωd=Mα1−Mα(2u0)2.

And so, for an estimate of ωd, call it ωd, this equation provides an estimate of Mα, call it Mα. That estimate and (E1.3a) results in the desired estimate of CLα, call it CLα.

Method 3. Use both decay and oscillation frequency information: This method is best described using a plot of (E1.6): [The log decrement method ]

Figure 1. Initial condition response for ωn=2πand ς=0.01.

The plot on the right in figure 1 shows that for t0=10 sec, θ(t0)≃5.4o, and for t1=20 sec, θ(t1)≃3o. The duration, t1−t0 between these times is 20 sec., but it is also 10 cycles (or periods), where one period is T=2πωd=2πωn1−ς2. Hence, t1−t0=20πωn1−ς2.

Now, from (6a), we have θ(t0)≃θ0 e−ςωnt0 and θ(t1)≃θ1 e−ςωnt1. And so

θ(t0)θ(t1)= e−ςωnt0 e−ςωnt1=e−ςωn(t1−t0)=e20π ς1−ς2.

Define δ=Δς1−ς2. Then we have δ=lnθ(t0)θ(t1)20π =ln(5.43)20π = 0.0094. From the definition of δ, we have ς=δ1−δ2. And so, our estimate δ= 0.0094 gives ς≃ 0.0094. This estimate is close to the true value ς=0.01, and it would have been closer, had we not used visually-based estimates of θ(t0)and θ(t1).

Finally, we can use (E1.3a), (E1.5b) and (E1.5c) to obtain:   CLα=8IyρSl ς2.

Notice that in this method it is not necessary to measure u0, since the above estimate does not involve it. This can offer a significant advantage over other methods, both in terms of accuracy and equipment.

(d) Use of the setup in Figure 4.9 to validate the experimental design: Now that we know how to use the transient response, (6) to estimate the lift coefficient derivative for a given tail design, it is necessary to validate the experimental setup illustrated in Figure 4.9. In that design, we are assuming that the cross-bar bearing friction is negligible. We will also assume that the tube that supports the tail is completely rigid, and that its aerodynamic influence is negligible. Finally, we will assume that we have perfect measurements of the geometric and mass quantities described in that figure. By using a flat plate with known lift properties, we can compute the theoretical value for CLα. If the experimentally measured transient response matches our theoretical prediction reasonably well, then we can assume that we have a valid setup. We will now proceed to compute the theoretical value for CLαfrom the given numerical information.

The moment of inertia, Iy: From the parallel axis theorem, we have: Iy=Iy'+mpl2.The moment of inertia of the flat plate about its cg, and the moment of the plate about the setup cg are:

Iy'=(112)ρbtc3=(112)(Wpg)c2=2.16×10−5slug−ft2   &   mpl2=(Wpg)l2=9.3×10−3slug−ft2

Hence, Iy(plate)=9.32×10−3slug−ft2. We also have Iy(Ballast)=4.60×10−3slug−ft2. Hence, the moment of inertia of the entire system is: Iy≃1.4×10−2slug−ft2.

Numerical values for Mα and Mq:

For an infinite flat plate, we have CLα(∞)=2πrad. For a finite plate with aspect ratio, AR, the lift coefficient derivative is: CLα=CLα(∞)[1+CLα(∞)(π AR)]. Since AR=6, we have CLα=4.7rad

Using these highlighted results and equations (3), we obtain: Mα=−36.1s2 and Mq=−1.38s2

Hence, (2) becomes:   θ+1.38 {θ+36.1θ=0. It follows the theoretical values for ωnand ςare:

ωn=6.0 rad/secand ς=0.038. If we give this plate an initial angular displacement, θ1=100, then we should expect the experimentally measured transient response to be similar to the plot below.

Figure 2. Expected description of the experimentally measured transient response of the flat plat to an initial angular displacement of 10o.   □

Example Problem 4.2 (Nelson p. 151) Given the differential equations:

x1+0.5x1−10x2=−1 δ ∧ x2−x2+x1=2δ

(a) Rewrite these in state space form x=Ax+Bη.

Solution: x1=−0.5x1+10x2−1 δ ∧ x2=x2−x1+2δ, or, x1x2=−0.510−11 x1x2+−12δ.

(b) Find the free response eigenvalues.

Solution:  The eigenvalues of A=−0.510−11  are the values of s that solve determinant equation |sI−A|=0. Specifically:

|sI−A|=s+.5−101s−1=(s+.5)(s−1)+10=s2−0.5s+9.5 =Δp(s)

The roots of the characteristic polynomial p(s) are: s1,2=0.25 ±i3.07.

(c) What do these eigenvalues tell us about the response of the system?

Answer: The real parts of the eigenvalues are positive, and so the system is dynamically unstable. Specifically, the response to an initial disturbance would grow exponentially. This envelop is of the form

e0.25 t. It is common to compute the time it takes for this amplitude envelope to double. We will now compute this time.

Let A(t1)=Ce0.25 t1, and A(t2)=Ce0.25 t2. Then A(t2)A(t1)=e0.25( t2−t1). For amplitude doubling, we have : e0.25( t2−t1)=2, or: t2−t1=4ln(2)= 2.77 seconds.

(d) Use of Laplace Transforms to obtain the eigenvalues of A:

For students who do not feel confident about matrix algebra, but who have background in the use of the Laplace transforms, here as an alternative solution method:

ℓ[x1=−0.5x1+10x2−1 δ] ⇒ sX1(s)=−0.5X1(s)+10X2(s)−1	(E2.1a)

ℓ[x2=x2−x1+2δ] ⇒ sX2(s)=X2(s)−X1(s)+2	(E2.1b)

Equations (E2.1) include two unknowns; namely X1(s)and X2(s). And so, either unknown can be solved for using purely algebraic operations. We will now solve these equations for each of these unknowns. These two resulting expressions will have something in common, as will be seen.

Solution of (E2.1) for X1(s):

From sX2(s)=X2(s)−X1(s)+2, we have X2(s)=−X1(s)+2s−1. Substituting this into (1a) gives:

sX1(s)=−0.5X1(s)+10−X1(s)+2s−1−1

s(s−1)X1(s)=−0.5(s−1)X1(s)+10[−X1(s)+2]−(s−1). Gathering terms gives:

(s2−0.5s++9.5)X1(s)=21−s.	(E2.2a)

The corresponding differential equation is:

x1−0.5x1+9.5x1=21δ−δ.	(E2.2b)

Solution of (E2.1) for X2(s):

From sX2(s)=X2(s)−X1(s)+2, we haveX1(s)=(1−s)X2(s)+2. Substituting this into (E2.1a) gives:

(s+0.5)[(1−s)X2(s)+2]=10X2(s)−1. Gathering terms gives:

(s2−0.5s+9.5)X2(s)=2s+2.	(E2.3a)

The corresponding differential equation is:

x2−0.5x2+9.5x2=2δ+δ.	(E2.3b)

Remark 1. Notice that the left sides of equations (E2.2) and (E2.3) are identical. In particular, the 2-D state space system characteristic polynomial, p(s)=s2−0.5s+9.5is common to both solutions. Hence, the system eigenvalues can be obtained from either solution.

Remark 2. This method of solving for the system eigenvalues is comparable in terms of mathematical manipulations to the matrix method associated with parts (a) and (b). However, recall that the longitudinal state space system includes four equations in four unknowns. In that situation, to solve for any chosen unknown would require much more work than what was required in this simple setting of two equations in two unknowns.

Remark 3. In this simple example, the Laplace transform method has the advantage of not only leading to the system eigenvalues, but to the solution for each chosen unknown. Specifically:

X1(s)=−s+21s2−0.5s++9.5       and         X2(s)=2s+2s2−0.5s+9.5

These are solutions in the Laplace (i.e. s domain). To obtain the corresponding solutions in the time (i.e. t domain), we can use a table of Laplace transform pairs.

[ e.g. http://en.wikipedia.org/wiki/Laplace_transform#Table_of_selected_Laplace_transforms ]

At this point in the course, we will not address this table. Instead, we will simply use Matlab to obtain plots of x1(t) and x2(t). The plots are:

Figure 1.  Plots of x1(t) (BLUE) and x2(t)(GREEN).

The code is:

%PROGRAM NAME: x12solutions.m

% This code computes plots of x1(t) and x2(t) via Laplace

p = [1 -0.5 9.5];

n1 = [-1 21];

n2 = [2 2];

sys1 = tf(n1,p);

sys2 = tf(n2,p);

impulse(sys1)

pause

hold on

impulse(sys2)

axis([0 20 -200 200])

grid

Discussion of the plots: Both responses ‘explode’. This is because the roots of the characteristic polynomial p(s)=s2−0.5s+9.5, which are s1,2=0.25 ±i3.07, have a positive real part (i.e. they are in the right half of the s plane). They both oscillate at the same frequency as they explode. However, it is clear that x1(t) is exploding faster than x2(t). This is because the right side of (2b) includes the term 21δ=21δ(t), where δ(t)is called the unit impulse. In more mathematical terms, it is also called the Dirac delta function. □

Approximation of the Phugoid and Short-Period Longitudinal Modes (Nelson p.152)

From p.7 of these notes, we have:

ΔuΔwΔqΔθ=XuXw0−gZuZwu00Mu+MwZuMw+MwZwMwu0+Mq00010 ΔuΔwΔqΔθ +XδZδMδ+MwZδ0 XδTZδTMδT+MwZδT0ΔδΔδT (1)

This dynamical system is 4-D, and so the characteristic polynomial will have four roots. These roots will occur as two pairs of complex-conjugate roots. Recall that a 2-D dynamical system with a pair of complex-conjugate roots will has a response that is oscillatory. It is characterized by a damping ratio, ς, and an undamped natural frequency, ωn. And so, the above 4-D system will include (ς1,ωn1)and (ς2,ωn2). One of the natural frequencies, say, ωn1will be a very low frequency (i.e. a very long period). This corresponds to what is known as the Phugoid, or long-period longitudinal mode. The other frequency, ωn2>>ωn1, is a high frequency mode, that is simply referred to as the short period longitudinal mode.

Ultimately, we will address the above 4-D dynamical system. That will necessitate matrix algebra. In this section we will make some simplifying assumptions that will allow us to address two separate 2-D systems; one related to the phugoid mode, and the other related to the short period mode. Not only will this alleviate the need for matrix algebra (i.e. using Laplace transforms), but it will highlight the variables that are most significant in relation to each mode.

The phugoid mode:  The phugoid mode as a long period mode. Typically, the period is on the order of many seconds (or even minutes). As noted by Nelson: “It represents a gradual interchange of potential and kinetic energy about the equilibrium attitude and airspeed.” Another description is:

“The phugoid has a nearly constant angle of attack but varying pitch, caused by a repeated exchange of airspeed and altitude. It can be excited by an elevator singlet (a short, sharp deflection followed by a return to the centered position) resulting in a pitch increase with no change in trim from the cruise condition. As speed decays, the nose will drop below the horizon. Speed will increase, and the nose will climb above the horizon. Periods can vary from under 30 seconds for light aircraft to minutes for larger aircraft. Microlight aircraft typically show a phugoid period of 15–25 seconds, and it has been suggested that birds and model airplanes show convergence between the phugoid and short period modes. A classical model for the phugoid period can be simplified to about (0.85 × speed in knots) seconds, but this only really works for larger aircraft. Phugoids are often demonstrated to student pilots as an example of the speed stability of the aircraft and the importance of proper trimming. When it occurs, it is considered a nuisance, and in lighter airplanes (typically showing a shorter period) it can be a cause of pilot-induced oscillation. The phugoid, for moderate amplitude,[1] occurs at an effectively constant angle of attack, although in practice the angle of attack actually varies by a few tenths of a degree. This means that the stalling angle of attack is never exceeded, and it is possible (in the <1g section of the cycle) to fly at speeds below the known stalling speed. Free flight models with badly unstable phugoid typically stall or loop, depending on thrust.[2] An unstable or divergent phugoid is caused, mainly, by a large difference between the incidence angles of the wing and tail. A stable, decreasing phugoid can be attained by building a smaller stabilizer on a longer tail, or, at the expense of pitch and yaw "static" stability, by shifting the center of gravity to the rear.”                      [	http://en.wikipedia.org/wiki/Phugoid ]

As such, we can make the following assumptions in relation to the above 4-D dynamical system:

(A1): The pitch moment dynamics are negligible. Mathematically, this means that we remove the third equation from (1) above. Note: In doing so, we must then replace Δqon the right side of (1) by what it actually is; namely, Δθ. This alleviates the need for the fourth equation, which is then also removed.

(A2): The vertical velocity Δw≃0 , and the vertical acceleration Δw≃0.

(A3): The control forces and moments are zero.

These assumptions result in the equations:

Δu=XuΔu − gΔθ     and     0=ZuΔu + u0Δθ.

Rearranging these in state space form gives the 2-D dynamical system:

ΔuΔθ=Xu−g−Zuu00 ΔuΔθ .	(2)

The system eigenvalues are found from:

sI−Xu−g−Zuu00  = s−XugZuu0s = s2−Xus−gZuu0 =0.

Hence,

s1,2=12Xu±Xu2+4gZuu0.	(3)

Problem 1. Explain why it must be that Xu2+4gZuu0<0.

Problem 2. Show that the phugoid undamped natural frequency is ωnp=−gZuu0.

Problem 3. From p.150 of Nelson, we have Zu=−(CLu+2CL0)QSmu0,  where Q=Δ12ρu02. Use this and the result in Problem 2 to determine the type of relation between the nominal speed, u0, and ωnp.

Problem 4. Nelson states on p.153 that “If we neglect compressibility effects,… then ωnp=2gu0.”

Explain how this compares to your answer in Problem 3.

Problem 5. In EXAMPLE PROBLEM 4.3 on p.155 (of Nelson) we have:

Q=12ρu02=(0.5)(.002378 slugft3)(176 fts)2 = 36.8 lbft2

QS=(36.8 lbft2)(184 ft2) = 6771 lb

Zu=−(CLu+2CL0)QSmu0 = −[0.0+2(.41)](6771 lb)(85.4 slugs)(176 fts) = −0.369 (s−1)

These figures are used on the bottom of p.158 for:

ωnp=−gZuu0 = (0.369)(32.21)176 = 0.26 rads.

Compute the numerical value of ωnpusing the above equations, but with u0=1. Then comment.

Solution: From p.150 of Nelson, we have:

Xu=−(CDu+2CD0)QSmu0 and Zu=−(CLu+2CL0)QSmu0,  where Q=Δ12ρu02 ,

The Short Period Mode:

Before we proceed with the material in Nelson, it is worth reviewing another development of this mode; one that may give the reader a greater appreciation than is given in Nelson, of the nature of this mode

From the posted notes “Wiki Flight Dynamics:

NOTE: I have highlighted points and added comments/remarks to emphasize items that I think are important.

A short input (in control systems terminology an impulse) in pitch (generally via the elevator in a standard configuration fixed wing aircraft) will generally lead to overshoots about the trimmed condition. The transition is characterized by a damped simple harmonic motion about the new trim. There is very little change in the trajectory over the time it takes for the oscillation to damp out.

Generally this oscillation is high frequency (hence short period) and is damped over a period of a few seconds. A real-world example would involve a pilot selecting a new climb attitude, for example 5º nose up from the original attitude. A short, sharp pull back on the control column may be used, and will generally lead to oscillations about the new trim condition. If the oscillations are poorly damped the aircraft will take a long period of time to settle at the new condition, potentially leading to Pilot-induced oscillation. If the short period mode is unstable it will generally be impossible for the pilot to safely control the aircraft for any period of time.

This damped harmonic motion is called the short period pitch oscillation, it arises from the tendency of a stable aircraft to point in the general direction of flight. It is very similar in nature to the weathercock mode of missile or rocket configurations. The motion involves mainly the pitch attitude θ (theta) and incidence α (alpha). The direction of the velocity vector, relative to inertial axes is θ − α. The velocity vector is:

uf = Ucos(θ − α)

wf = Usin(θ − α)

where uf,wf are the inertial axes components of velocity. According to Newton's Second Law, the accelerations are proportional to the forces, so the forces in inertial axes are:

where m is the mass. By the nature of the motion, the speed variation is negligible over the period of the oscillation, so:

But the forces are generated by the pressure distribution on the body, and are referred to the velocity vector. But the velocity (wind) axes set is not an inertial frame so we must resolve the fixed axes forces into wind axes. Also, we are only concerned with the force along the z-axis:

Z = − Zfcos(θ − α) + Xfsin(θ − α)

Or:

In words, the wind axes force is equal to the centripetal acceleration.

The moment equation is the time derivative of the angular momentum:

where M is the pitching moment, and B is the moment of inertia about the pitch axis. Let: , the pitch rate. The equations of motion, with all forces and moments referred to wind axes are, therefore:

We are only concerned with perturbations in forces and moments, due to perturbations in the states α and q, and their time derivatives. These are characterized by stability derivatives determined from the flight condition. The possible stability derivatives are:

Zα Lift due to incidence, this is negative because the z-axis is downwards whilst positive incidence causes an upwards force.

Zq Lift due to pitch rate, arises from the increase in tail incidence, hence is also negative, but small compared with Zα.

Mα Pitching moment due to incidence - the static stability term. Static stability requires this to be negative.

Mq Pitching moment due to pitch rate - the pitch damping term, this is always negative.

Since the tail is operating in the flowfield of the wing, changes in the wing incidence cause changes in the downwash, but there is a delay for the change in wing flowfield to affect the tail lift, this is represented as a moment proportional to the rate of change of incidence:

Increasing the wing incidence without increasing the tail incidence produces a nose up moment, so is expected to be positive.

The equations of motion, with small perturbation forces and moments become:

These may be manipulated to yield a second order linear differential equation in α:

This represents a damped simple harmonic motion.

We should expect to be small compared with unity, so the coefficient of α (the 'stiffness' term) will be positive, provided . This expression is dominated by Mα, which defines the longitudinal static stability of the aircraft, it must be negative for stability. The damping term is reduced by the downwash effect, and it is difficult to design an aircraft with both rapid natural response and heavy damping. Usually, the response is underdamped but stable.

Nelson’s Development:

We repeat the 4-D longitudinal state space system here for convenience:

ΔuΔwΔqΔθ=XuXw0−gZuZwu00Mu+MwZuMw+MwZwMwu0+Mq00010 ΔuΔwΔqΔθ +XδZδMδ+MwZδ0 XδTZδTMδT+MwZδT0ΔδΔδT (1)

On p.154, Nelson states: “An approximation to the short period mode of motion can be obtained by assuming Δu=0, and dropping the X-force equation. The longitudinal state space equations are reduced to the following:

ΔwΔq = Zwu0Mw+MwZwMwu0+Mq ΔwΔq.	(2)

Potential Exam 2 Problem: Recall the assumptions used to arrive at (2) above:

(A1) Assume thatΔu=0

(A2): The X-force equation can be dropped.

(a) Write the 4-D dynamical system (1) for these two assumptions:

Solution:

ΔuΔwΔqΔθ=000−gZuZwu00Mu+MwZuMw+MwZwMwu0+Mq00010 0ΔwΔqΔθ +0ZδMδ+MwZδ0 0ZδTMδT+MwZδT0ΔδΔδT

(b) Write the first equation of this system, and explain what it relates to.

Solution: The equation is: Δu=−g Δθ. This is equivalent to: mΔu=−mg Δθ, which is simply Newton’s second law in the xE direction for sinΔθ≃Δθ.

(c) Write the fourth equation of this system, and describe it in words.

Solution: The equation is: Δθ=Δq. In words, this says that the rate of change of the second Euler angle is equal to the pitch rate.

(d) Since we have ‘extracted’ the first and fourth equations in parts (b) and (c), we can ‘zero-out’ these rows in the 4-D system of equations. Write this 4-D system with the first and fourth rows zeroed out. Circle the elements in the A-matrix that no longer matter. Then discuss how this 4-D system compares to the 2-D system (2) above.

Solution:

0ΔwΔq0=0000ZuZwu00Mu+MwZuMw+MwZwMwu0+Mq00000 0ΔwΔq0 +0ZδMδ+MwZδ0 0ZδTMδT+MwZδT0ΔδΔδT

Comment: This simplified 4-D system is similar to the 2-D system, (2), except that it retains the w- and q- control forces. □

On p.154, Nelson then proceeds to reformulate the 2-D system, (2), in relation to Δα, as opposed to Δw:

Δα=Δwu0.	(3)

Explain why the equality (3) is valid:

Explanation: __________________________________________________________________________

_____________________________________________________________________________________

From (3), (2) becomes:

u0ΔαΔq = Zwu0Mw+MwZwMwu0+Mq u0ΔαΔq

⇒ u0ΔαΔq = u0Zwu0u0(Mw+MwZw)Mwu0+Mq ΔαΔq

⇒ 1u0001 u0ΔαΔq = 1u0001 u0Zwu0u0(Mw+MwZw)Mwu0+Mq ΔαΔq

⇒ ΔαΔq = Zw1u0(Mw+MwZw)Mwu0+Mq ΔαΔq.	(4)

Recall that in Nelson, the definition of Mαis: Mα=Δ1Iy×∂M∂α. From (3) above, we have :

∂α=∂wu0. And so, Mα=Δu0Iy×∂M∂w = u0 Mw. In a similar way, one can show that Zα=u0Zw  and   Mα=u0Mw. With these results, (4) can be re-written as:

ΔαΔq = Zαu01Mα+MαZαu0Mα+Mq ΔαΔq.	(5)

The characteristic polynomial for the 2-D dynamical system, (5) is:

p(s)=s2−Mq+Mα+Zαu0s+MqZαu0 − Mα.	(6)

We do not need to compute the roots of (6) in order to extract valuable information in relation to (5).

Consider the polynomial: p(x)=x2+bx+c. The roots of this polynomial are

x1,2=0.5−b±b2−4c.

Case 1: real roots   (i.e.b2−4c>0)

QUESTION 1: Under what condition(s) will both roots be negative?

Answer: _______________________________________________________________________

QUESTION 2: In relation to (6), why is QUESTION 1 important?

Answer: _______________________________________________________________________

QUESTION 3: Assume that b,c>0. Give the expression for the root that is closest to zero.

Answer: ___________________________________________________________________

Question 4: What is the significance of the root given in response to QUESTION 3, in relation to (6)?

Answer: ___________________________________________________________________

Case 2: complex conjugate roots   (i.e.b2−4c<0)

QUESTION 1: Under what condition(s) will both roots have negative real parts?

Answer: _______________________________________________________________________

QUESTION 2: Is it possible that we can have c<0for this case? Explain.

Answer: _______________________________________________________________________

QUESTION 3: Assume that b,c>0. Explain the implications of this case in relation to (6).

Answer: ________________________________________________________________________

QUESTION 4: For the case b,c>0, write p(s)=s2+bs+c as p(s)=s2+2ςωns+ωn2. Give the expressions for b and c in terms of ς and ωn.

Answer: ______________________________________________________________________

Conclusions in Relation to (6):

QUESTION 1: Under what conditions will (6) be a stable system?

Answer: ______________________________________________________________________

QUESTION 2: Assuming (6) is a stable system, under what condition(s) will it exhibit an oscillatory response to an initial condition?

Answer: _________________________________________________________________________

QUESTION 3: Assuming that (6) is stable, and exhibits oscillatory behavior, give expressions for ς and ωnin terms of the various parameters in (6).

Answer: _________________________________________________________________________

PROBLEM Investigate how ς and ωnrelate to the plane nominal speed, u0.

Aircraft Lateral Dynamics  [Nelson Chapter 5 p.181]

1. Pure Rolling Motion: [Section 5.2 on p.182] Wind tunnel tests are often used to estimate roll moment parameters. Consider a plane that is mounted in a wind tunnel in a way that it is constrained to pure roll motion about the x-axis. Newton’s second law gives:

Mx=IxΔφ = ∂L∂δa∂δa=0Δδa + ∂L∂pp=0Δp.	(1a)

Here, Δδais a small deflection of the ailerons, and Δpis a small perturbation of the roll rate. Since the plane cg is constrained, we have:

Δp = Δφ.	(1b)

Equations (1) result in:

IxΔp − ∂L∂pp=0Δp = ∂L∂δaδa=0Δδa.	(2)

Before we continue our development in relation to (2), it is worth spending a little time to discuss the properties of a first order constant coefficient differential equation from a systems perspective.

The structure of a first order dynamical system:

From (2), we see that the dynamics of pure roll correspond to a first order dynamical system. One standard form for such a system is:

τy(t) + y(t) = gsf(t). 	(3a)

The transfer function associated with (3a) is obtained by taking its Laplace transform (under zero initial conditions!). Doing this gives:

τsY(s) + Y(s) = gsF(s).	(3b)

The system transfer function is then:

G(s) =Δ Y(s)F(s) = gsτs+1.	(3c)

It follows that, for a given ‘input’, f(t), we can find the ‘response, y(t), via a table of Laplace

transform pairs, in relation to:

Y(s) = G(s)F(s) = gsτs+1F(s).	(3d)

Use of a Table of Laplace Transforms:

The system unit impulse response: From the table below, entry #1 shows that the Laplace transform of the unit impulse, f(t)=δ(t)is F(s)=1. For this input, (3d) becomes:

Y(s) = G(s) = gsτs+1 = gsτs+1τ.

Again, using entry #4, we obtain:

y(t) = g(t) = gsτ e−tτ.	(3e)

The system unit-step response: From the table below, entry #2 shows that the Laplace transform of the unit step, f(t)=u(t)is F(s)=1s. For this input, (3d) becomes:

Y(s) = G(s)s = gsτs(s+1τ).

Again, using entry #7, we obtain:

y(t) = gs(1− e−tτ).	(3f)

Remark 1. Both the unit impulse and the unit step response include the term e−tτ. Notice that this term is a consequence of the fact that the system characteristic polynomial, p(s)=τs+1, has a single root: s1=−1τ. And so, es1t=e−tτ. This term will decay to zero as t→∞if τ>0, or equivalently, if the root, s1=−1τ, is in the Left Half (of the complex) Plane (LHP). Moreover, for t=4τit will have the value e−4ττ=e−4≃0.02. The quantity 4τis commonly termed the “4τ” or “2%” settling time.

Remark 2. From (3f), the asymptotic response to the unit step input is:

yss=Δlimt→∞y(t) = limt→∞gs(1− e−tτ) = gs.	(3g)

Since the input has amplitude 1.0 and the response has steady state amplitude gs, the ratio of these amplitudes is simply gs. For this reason, this ratio is called the system static gain.

Summary: A first order system of the type described by (3a) is characterized by two parameters: the system time constant, τ, and the system static gain, gs.     □

# Table of Laplace and Z Transforms

(Please email me if you find an error) Using this table for Z Transforms with Discrete Indices
Shortened 2-page pdf of Laplace Transforms and Properties

| Entry # | Laplace Domain | Time Domain | Z Domain (t=kT) |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 |  |  |  |
| 14 |  |  |  |
| 15 |  |  |  |
| 16 |  |  |  |

We are now in a position to gain some insight into (2) without much effort. Re-writing it in the ‘standard form’ (3a) gives:

Ix−∂L∂p−1Δp + Δp = ∂L∂δa−∂L∂p−1Δδa.	(4a)

The system time constant and static gain are:

τ = Ix−∂L∂p−1     and     gs = ∂L∂δa−∂L∂p−1.	(4b)

Some observations-

(O1): Stability requires that τ>0. In other words, we must have ∂L∂p|Δp=0 <0.

(O2): The 4τresponse time is proportional to Ix, and inversely proportional to |∂L∂p|.

(O3): Since the input, Δδa(t) has units of radians, and the output, Δp(t), has units of radians/sec., the static gain, gs, has units rad/secrad. Some people would write this simply as 1sec. But this misses the point that gsis a ratio.

(O4): The static gain, gs, is directly proportional to ∂L∂δa, and inversely proportional to ∂L∂p

Notation: In Nelson’s book, he defines Lp=Δ(∂L∂p)Ix and Lδa=Δ(∂L∂δa)Ix.

With this notation, (4) becomes:

(−1Lp)Δp + Δp = (−LδaLp)Δδa.	(5a)

The system time constant and static gain are:

τ = −1Lp     and     gs = −LδaLp.	(5b)

EXAMPLE PROBLEM 5.1 (Nelson p.183)

Calculate the roll response of an F104A aircraft to a 5o step change in aileron deflection. Assume the plane is flying at sea level with a velocity of u0=87 ms.

Clp=−0.285 rad−1 ; Clδa=0.039 rad−1 ; S=18 m2 ; b=6.7 m ; Ix=4676 kg−m2

Solution: The F104A has the following aerodynamic and geometric characteristics:

.

Hence, Q =Δ0.5ρu02 = 4636 Nm2 ; b2u0=0.039 s. And so:

Lp=Clp(b2u0)QSbIx = −1.3 s−1 ; Lδa=ClδaQSbIx = 4.66 s−2.

From (5b) above, we obtain:

τ = 0.77 s     and     gs = −3.5846 (rs)r.

For the aileron input Δδa=5o=5(π180)=0.0873 r, the roll response will achieve a steady state roll rate of Δpss=0.3129 rs after approximately 4τ=3.08 s. From (3f), the mathematical form of this step response is: y(t) = −3.5846(5o)(1− e−t0.77). This response is plotted below.

Figure 1. Plot of the F104A roll rate response to a 5o step aileron input.

2. Pure Yawing Motion:  [Section 5.3 on p.188]

Wind tunnel tests are often used to estimate yaw moment parameters. Consider a plane that is mounted in a wind tunnel in a way that it is constrained to pure yaw motion about the z-axis. Newton’s second law gives:

Mz=IzΔψ=∂N∂βΔβ+∂N∂βΔβ+∂N∂rΔr+∂N∂δrΔδr.	(6a)

In general, Δδris a small deflection of the rudder, Δris a small perturbation of the yaw rate, Δβis a small perturbation of the sideslip angle, and Δβis a small perturbation of the sideslip rate. Since the plane cg is constrained, we have:

Δψ=−Δβ     and     Δψ=−Δβ=Δr	(6b)

Equations (6) result in:

Δψ + (Nβ−Nr)Δψ + NβΔψ = NδrΔδr	(7)

where we have defined N=Δ(∂N∂∗)Iz. Since it is known that the response to a rudder input, Δδr, will be oscillatory and decaying in nature, (7) represents a second order underdamped dynamical system. The standard form for such a system is:

Δψ + 2ςωnΔψ + ωn2Δψ = (gsωn2)Δδr.	(8a)

Equating (7) and (8a) we immediately have the following:

ωn=Nβ ,   ς=Nβ−Nr2Nβ,     τ=(ςωn)−1=2(Nβ−Nr)  and  gs=(NβNδr)−1.	(8b)

The damped natural frequency is:

ωd=ωn1−ς2=Nβ1−Nβ−Nr2Nβ2 = 124Nβ−(Nβ−Nr)2.	(8c)

Hence, the damped natural oscillation period is:

Td=2πωd= π4Nβ−(Nβ−Nr)2.	(8d).

Remark 1. Nelson notes that the term Nβis usually negligible. In this case, we have:

ωn=Nβ; ς=Nr2Nβ; τ=2Nr;gs=NβNδr;ωd= 124Nβ−Nr2;Td= π4Nβ−Nr2.

Remark 2. On p.190, Nelson give plots of Δβ(t)ΔβTRIM versus t for various values of ς. This ratio converges to 1.0 for every ς. The problem is: the term ΔβTRIMis never defined. To figure out what it is, we need only look at the system static gain: gs=NβNδr. Since limt→∞Δβ(t)=Δ Δβss=ΔβTRIM=gsΔδr, we have determined the expression for ΔβTRIM:  ΔβTRIM=gsΔδr.

EXAMPLE PROBLEM 5.2 Suppose an airplane is constrained to a pure yawing motion. Use the data for the general aviation airplane in Appendix B, determine the following quantities:

(a) The yaw moment equation written in state space form.

(b) The characteristic equation and eigenvalues for the system.

(c) The damping ratio, ς, and undamped natural frequency, ωn.

(d) The response of the plane to a 5o rudder input. Assume initial conditions are: Δβ(0)=Δr(0)=0.

Solution: For sea level flight condition, the weathercock stability coefficient, the yaw damping coefficient, and the rudder control power coefficient have, respectively, the following values:

Cnβ=0.071 r ; Cnr=−0.125 r ; Cnδr=−0.072 r.

[The derivative Cnβis not included in the appendix, and will be assumed to be zero.]

For u0=176 fts, the dynamic pressure at sea level is: Q =Δ0.5ρu02 = 36.8 lbft2.

The plane geometry parameters include: S=184 ft2 ; b=33.4 ft ; Iz=3530 slug−ft2.

These values result in the following dimensional derivative values:

Nβ=Cnβ(QSbIz)=4.55 s2 ; Nr=Cnr[(b2u0) QSbIz]=−0.76 s; Nδr=Cnδr(QSbIz)=−4.6 s2

Substituting these into (7) gives:

Δψ + 0.76Δψ + 4.55Δψ = −4.6Δδr. 	(1)

We are now in a position to solve parts (a-d).

(a) The yaw moment equation written in state space form.

Solution: Recall that the state space form is: x=Ax+Bu. In relation to (1) above, this will be a 2-D system. Hence, x(t)=[x1(t) x2(t)]tr We know that one state, say, x1(t), will be Δψ(t). We also know that, since the plane is constrained to pure yawing, Δψ(t)=Δr(t). And so, let x2(t)=Δr(t). From (1) we have: Δr + 0.76Δr + 4.55Δψ = −4.6Δδr. And so, we arrive at:

ΔrΔψ = −0.76−4.5510 ΔrΔψ +−4.60 Δδr .

(b) The characteristic equation and eigenvalues for the system.

Solution: The characteristic polynomial is simply: p(s)=s2 + 0.76s + 4.55. The eigenvalues of A = −0.76−4.5510 are obtained via Matlab:

> A=[-.76 -4.55 ; 1 0];

> eig(A)

ans =

-0.3800 + 2.0990i

-0.3800 - 2.0990i

They can also be obtained via:

> p=[1 .76 4.55];

> roots(p)

ans =

-0.3800 + 2.0990i

-0.3800 - 2.0990i

(c) The damping ratio, ς, and undamped natural frequency, ωn.

Solution: These are easily computed: ωn=2.13 rs ; ς=0.178

(d) The response of the plane to a 5o rudder input. Assume initial conditions are: Δβ(0)=Δr(0)=0.

Solution: A homework problem?    □

The Spiral mode Approximation: [Section5.4.1 on p.195]

This mode is characterized by changes in the bank angle, φ, and the heading angle, ψ. An approximation to this mode can be obtained from the above 4-state system by neglecting the side force equation and Δφ. Consequently, he obtains the following:

LβΔβ+LrΔr=0     and     Δr=NβΔβ+NrΔr.

These result in the following first order system description:

Δr+LrNβ−LβNrLβΔr=0

The characteristic polynomial is: p(s)=s+LrNβ−LβNrLβ.

The system root is: s1=−LrNβ−LβNrLβ.

The time constant is: τ=Lβ(LrNβ−LβNr).

The Roll Approximation: [Section 5.4.2 on p.198]

This motion is approximated by pure roll. Hence, from p.4.32 of these notes (neglecting inputs), we have:

Δp − LpΔp = 0.

The characteristic polynomial is: p(s)=s−Lp.

The system root is: s1=Lp.

The time constant is: τ=−1Lp.

Dutch Roll Approximation [Section 5.4.3 on p.198]

As noted by Nelson, the Dutch roll is characterized primarily by sideslipping and yaw. (i.e. the rolling moment equation can be ignored). As such, the 4-D lateral system becomes the 2-state system:

ΔβΔr = Yβu0−1−Yru0NβNr ΔβΔr +0Yδru0NδaNδr ΔδaΔδr .

Since it is known that this mode includes oscillations, this is a second order underdamped system.

Example: Conduct the following investigation of the Dutch roll dynamics.

(a) Compute the system characteristic polynomial directly from p(s)=|sI−A|.

Solution:

|sI− Yβu0−1−Yru0NβNr | = s−Yβu01−Yru0−Nβs−Nr =s−Yβu0(s−Nr)+Nβ1−Yru0 =p(s)

or                                       p(s) =s2−Yβ+u0Nru0s+YβNr+u0Nβ−YrNβu0

(b) Assume that the dynamics include natural oscillations (i.e. the system is stable, but underdamped). Compute expressions for the parameters associated with these dynamics.

Solution:  Equating the coefficients of p(s)with p(s)=s2+2ςωns+ωn2 gives:

ωn=YβNr+u0Nβ−YrNβu0      and      τ=(ςωn)−1=−2u0Yβ+u0Nr.

Since ς2ωn2=τ−2, we have ς2=τ−2ωn2, or ς2=(τωn)−2. Hence, the damping ratio is:

ς=−2u0Yβ+u0NrYβNr+u0Nβ−YrNβu0−1.

We could continue to derive expressions for ωdand for Td=2πωd, but we will withhold those for now.

(c) The expressions for the lateral directional derivatives related to the above are:

Yβ=Cyβ(QSm); Nβ=Cnβ(QSbIz); Yr=Cyr(QSb2mu0); Nr=Cnr(QSb22Izu0).

Recall that Q =Δ0.5ρu02 . Investigate the dependence of the above dynamic parameters as a function of the plane velocity, u0. [To be assigned as a homework problem.]

(d) Recall from EXAMPLE PROBLEM 5.3:

Nβ=Cnβ(QSbIz)=4.55 s2 and Nr=Cnr[(b2u0) QSbIz]=−0.76 s.

From Table B1. we have: Cyβ=−.564. Since there is no entry for Cyr, we will assume Cyr=0.

Hence, we have: Yβ=Cyβ(QSm)=−45.72 ∧ Yr=0. The characteristic polynomial is then:

p(s)=s2+1.102s+4.71. This yields ωn=2.17 rs and  ς=0.254. From these figures, give a description of the transient response of Δβ.

Solution: The time constant is τ≃1.8 sec.The damped natural frequency is ωd≃2.1 rs, and so the damped natural period is Td≃3 sec. Hence, the transient response will decay in about 7 seconds, during which time there will be only about 2.5 periods of oscillation.

(e) Verify your answer in (d) by plotting the impulse response corresponding to a transfer function H(s)=ωn2p(s).

Solution:  The Matlab commands needed to produce the plot are:

> p=[1 1.102 4.71];

> sys=tf(4.71,p)

Transfer function:

4.71

--------------------

s^2 + 1.102 s + 4.71

> impulse(sys)

> grid

(f) Explain why the above analysis did not provide any information about the system static gain.

Answer: In class. Since no defined input, no output, no static gain. 	□

The Full State Lateral Equations:    [Section 5.4 on p.193]

The lateral rigid body equations are found in Table 3.2 on p.108 and on p.193.

ddt−YvΔv−YpΔp+(u0−Yr)Δr−(gcosθ0)Δφ = YδrΔδr

−LvΔv+ddt−LpΔp−IxzIxddt+LrΔr = LδaΔδa+LδrΔδr               (Table 3.2)

−NvΔv −IxzIzddt+NpΔp + ddt−NrΔr = NδaΔδa+NδrΔδr

ΔvΔpΔrΔφ = AlatΔvΔpΔrΔφ + 0Lδa+IxzIxNδaNδa+IxzIzLδa0 YδrLδr+IxzIxNδrNδr+IxzIzLδr0ΔδaΔδr

where

Alat=YvYpYr−u0gcosθ0Lv+IxzIxNvLp+IxzIxNpLr+IxzIxNr0Nv+IxzIzLvNp+IxzIzLpNr+IxzIzLr00100.

The starred derivatives are defined as the un-starred derivatives divided by 1−Ixz2(IxIz).

If the product of inertia Ixz=0, these equations reduce to:

ΔvΔpΔrΔφ = AlatΔvΔpΔrΔφ + 0LδaNδa0 YδrLδrNδr0ΔδaΔδr

where

Alat=YvYpYr−u0gcosθ0LvLpLr0NvNpNr00100

The author notes that it is sometimes convenient to use the sideslip angle Δβ instead of the side velocity Δv. Since Δβ≃tan−1Δvu0≃Δvu0, the above equations become (for the product of inertia Ixz=0):

ΔβΔpΔrΔφ = AlatΔβΔpΔrΔφ + 0LδaNδa0 Yδru0LδrNδr0ΔδaΔδr

where

Alat=Yvu0Ypu0(Yru0)−1gcosθ0u0LvLpLr0NvNpNr00100

EXAMPLE PROBLEM 5.3

Consider once again, the general aviation aircraft (Table B.1on p.400 of Nelson).

(a) Find the eigenvalues of the 4-D system.

Solution: The state transition matrix is:

Alat=Yvu0Ypu0(Yru0)−1gcosθ0u0LvLpLr0NvNpNr00100=−.250−1.18−16.02−8.402.1904.49−.35−.7600100.

Using the Matlab command ‘eig(A)’ gives:

s1=  -8.4322      ;      s2= -0.4845 + 2.3329i       ;    s3=-0.4845 - 2.3329i    ;     s4=-0.0088

The spiral mode approximation is: s=−LrNβ−LβNrLβ=−0.144 s   .

The roll mode approximation is: s=Lp−8.44 s.

The Dutch roll approximation is obtained from p(s) =s2−Yβ+u0Nru0s+YβNr+u0Nβ−YrNβu0 using the quadratic formula, and is: s2,3=−0.51±2.11i.

Questions:

(Q1): Which modes correspond to which eigenvalues?

(Q2): Which approximation is the worst, and why?

(Q3): How good is the Dutch roll approximation re: the various dynamic parameters?         □

The Role of the Eigenvectors of A

1. Introduction

The eigenvalues of A determine the type of response of the system x=Ax+Buto both an input u(t)and to an initial condition x(0−)=Δx0. A real eigenvalue relates to an exponential type of response with no natural oscillations, whereas a conjugate pair of eigenvalues relates to an exponential type of response, but with natural oscillations at the damped natural frequency. The question that is addressed in this section is:

In what way(s) do the eigenvectors contribute to the response of the system x=Ax ; x(0−)=x0.

Notice that this question restricts attention to the initial condition response. As will be seen, even with this restriction, the answer to this question is mathematically nontrivial. These noted were compiled because I feel that, while both Etkin and Nelson give some insight as to how the eigenvectors influence the transient response, it is vague. These notes are an attempt to reduce that vagueness. They were compiled from the book Linear Systems by Thomas Kailath.

2. A Laplace Transform Approach

The Laplace transform of the n-D system

x=Ax ; x(0−)=x0	(1)

gives the Laplace domain solution

X(s) = (sI−A)−1x0.	(2)

In these notes we will assume that the eigenvalues of A, call them skk=1nare distinct (i.e. there are no repeated eigenvalues). Using the method of partial fraction expansions (a method that will be covered in AerE331), (2) can be written as:

X(s) = k=1n(s−sk)−1Rkx0.	(3)

The n×n matrix Rk is called the residue (matrix) associated with the eigenvalue sk.

The eigenvalue, sk , of the n×n system matrix Ais a scalar that satisfies the equation

Apk=skpk.	(4)

The column vector, pk, is called the eigenvector that corresponds to the eigenvalue sk. To be more precise, it is the right eigenvector that corresponds to the eigenvalue sk. Similarly, we have

qktrA=skqktr.	(5)

Here, the row vector qktris called the left eigenvector that corresponds to the eigenvalue sk. [Note the the superscript tr refers to the transpose. Hence, qkis an n×1column vector, and its transpose, qktr, is a 1×nrow vector. We now present the first of a number of ‘facts’ that will be given. The interested reader may refer to the book by Kailath for elaborations and/or proofs.

Fact 1:  (i) pkqktr = Rk     ;     (ii) qktrpk=1.

From (i), (3) becomes:

X(s) = k=1n(s−sk)−1(pkqktr)x0 = k=1n(s−sk)−1(qktrx0) pk =Δ k=1n(s−sk)−1αkpk 	(6)

where we have defined αk=Δqktrx0. The inverse Laplace transform of (6) is then:

x(t) = k=1nαkesktpk      where     αk=Δqktrx0.	(7)

Definition 1. The kth mode of (1) is x(k)(t)=αkesktpk.

It follows that (7) is composed of the sum of the n modes x(k)(t)k=1n. From (7), we arrive at one answer to the question in the Introduction of these notes:

(A1): The temporal shape of the kth mode of (7) is controlled solely by the eigenvalue, sk. All of the components of the n×1response vector x(t)have the same general temporal shape.

%PROGRAM NAME: ex43.m

% Nelson Example 4.3 on p. 155

% In the eqn Xdot = Ax, x = [ u w q th] on p.149

% Phugoid mode: xph = [u th]

%Short Period mode: xsp = [w q];

%==================SPECIFICATIONS=====================================

% Specify initial condition:

x0 = [10 0 0 0]';

%Specify time increment and total observation time:

%NOTE: Phugoid is ~100 sec. and Short period is ~1 sec.

dt = .001; tmax = 100;  tvec = 0:dt:tmax; lt = length(tvec);

% Specify A-matrix

A=[-.045 .036 0 -32.2;-.37 -2.02 176 0;.002 -.04 -2.95 0;0 0 1 0];

% ===== 4-D SYSTEM COMPUTATIONS USING EQN.7 on p.48 OF Ch4 NOTES =====

% Compute eiganvalues, and right (col.) and left (row) eigenvectors:

s = eig(A);

display('4-D system eigenvalues:')

disp(s)

pause

[P,D]=eig(A);

Q = P^-1;

% Compute {a(k)}k=1:4 where a(k)=alpha(k) = q(k)tr*x0:

a = Q*x0;

% Compute 4-D initial condition response

x = zeros(4,lt);

for j = 1:lt

for k=1:4

x(:,j) =x(:,j)+  a(k)*exp(s(k)*(j-1)*dt)*P(:,k);

end

end

% Plot real and imaginary parts to verify correctness (i.e imag. parts =0)

for m = 1:4

figure(m)

ur = real(x(m,:)); ui = imag(x(m,:));

plot(tvec,ur,tvec,ui)

xlabel('Time (sec)')

title('Figure # = Initial Condition Response #')

pause

end

%========== 2-D SYSTEM APPROXIMATION COMPUTATIONS =======================

%Compute Phugoid mode u(t) Response (i.e. state #1 of 4-state response

Xu=-.045; Zu=-.369; u0=176; g=32.2; uic=x0(1);

sys = tf(uic*[1 -Xu],[1 -Xu -g*Zu/u0]);

sph2 = roots([1 -Xu -g*Zu/u0]);

display('2-D phugoid eigenvalues:')

disp(sph2)

pause

figure(1)

hold

impulse(sys,tvec,'r')

title('u(t) phugoid response from 4-D Model (BLUE) & 2-D approx.(RED)')

pause

% Compute Short Period mode Response

Zw=-2.02;Mq=-2.05; Mw=-.05; Mwdot=-.0051;wic=x0(2);

a11=Zw; a12=u0; a21=Mw+Mwdot*Zw; a22=Mq+Mwdot*u0;

den = [1 -(a11+a22) (a11*a22-a12*a21)];

num = [1 -(a11+a22)];

sys1 = tf(num,den);

ssp2 = roots(den);

display('2-D short period eigenvalues:')

disp(ssp2)

pause

figure(2)

hold

impulse(sys1,tvec,'r')

title('w(t) short period response from 4-D Model(BLUE) & 2-D approx.(RED)')

pause

%=========================================================================

%=========================================================================

% THE FOLLOWING CODE WAS DEVELOPED TO VERIFY EQN.(7) OF CH4 NOTES VIA

%          THE MATLAB FUNCTION: 'ss'= state space

%-------------------------------------------------------------------------

% Compute TF matrix: G(j,k) = output(j)/input(k)

C=eye(4);

sys4 = ss(A,[],C,[]);

g = initial(sys4,x0,tvec);

figure(1)

plot(tvec,g(:,1),'k--','LineWidth',2)

title('u(t) phugoid response:eqn7 (BLUE),2-D approx.(RED),Matlab (BLACK')

Homework 7 PROBLEM 1: Use the above code to investigate the coupling between the four initial condition response variables, x(t)=[Δu(t) Δw(t) Δq(t) Δθ(t)]tr for initial conditions having only one nonzero entry. Recall that the phugoid mode uses the approximate 2-D variable x(ph)(t)=[Δu(t) Δθ(t)]tr, while the short period mode uses the approximate 2-D variable x(sp)(t)=[Δw(t) Δq(t)]tr.

Solution:

To begin this homework, we will investigate the initial condition: x0=[10 0 0 0]tr. This plane velocity disturbance will primarily excite the phugoid mode. Since we know that this mode has a time constant τ≃23 sec., we will specify the observation time to be 100 sec. The initial condition and observation time items in the Matlab code are highlighted in yellow. Running this code results in the following figures:

Figure 1. Plots of Δu(t)associated with the 4-D system (BLUE & BLACK) and with the 2-D phugoid approximation (RED), for initial condition x0=[10 0 0 0]tr.

Discussion: From the code, we have:

4-D system eigenvalues:

-2.4905 + 2.6110i

-2.4905 - 2.6110i

-0.0170 + 0.2152i

-0.0170 - 0.2152i

2-D phugoid eigenvalues:

-0.0225 + 0.2589i

-0.0225 - 0.2589i

The most obvious difference relates to the oscillation period T(ph)=2πωd sec.For the 4-D system this is T4D(ph)≃29 sec. For the 2-D approximation, it is T2D(ph)≃24 sec. This represents an error of -17%.

Figure 2. Plots of Δw(t)associated with the 4-D system (BLUE) and with the 2-D short period approximation (RED), for initial condition x0=[10 0 0 0]tr.

Discussion: Since the initial condition is zero for C and Δq(t), the 2-D approximation of the short period response will be zero. Figure 2 shows that, not only is the 4–D response not zero, it contains a short period response and a long period response. The latter has the same temporal structure as the phugoid response. This plot illustrates a ‘cross-coupling’ between these two modes.

Recall that the relation between Δw(t) and Δα(t) is approximately Δα(t)=Δw(t)u0. From this relation, we can get an idea of how this cross-coupling influences the angle of attack. Specifically, for Δw=0.5 ft/sec and u0=176 ft/sec, we have Δα≃0.16o. Hence, we can conclude that this cross-coupling will probably not be noticeable.

Remark: There are other figures included in the code. Also, while the code validates equation (7) on p.48 of these notes for Δu(t)[i.e. the black and blue lines match exactly in Figure 1 above], we have not validated it for the other three state variables, nor for other initial conditions. Hence, there are many things that one could continue to pursue in this investigation. 	□

Optional Extra Credit (80pts) : Repeat the investigation of PROBLEM 1 in relation to EXAMPLE PROBLEM 5.3 of Nelson (see p.44 of these notes). Due <= 12/2.

A Contribution from Kyle Litzer:

Phugoid Mode - http://www.youtube.com/watch?v=DEOGM_9NGTI&feature=related
Short Period Mode - http://www.youtube.com/watch?v=1O7ZqBS0_B8&NR=1

Pure Roll Mode - http://www.youtube.com/watch?v=zZQWI2QVwOg
Dutch Roll Mode - http://www.youtube.com/watch?v=oLe8ajpGNTs&feature=related
Spiral Mode - http://www.youtube.com/watch?v=3PFpHDD7rWY

On Thu, Nov 10, 2011 at 6:50 PM, Kyle Litzer <kjlitzer@iastate.edu> wrote:

Professor Sherman,

I found some videos on youtube that demonstrate (via flight simulator) the longitudinal and lateral aircraft dynamic modes. Aside from being pretty cool, I feel these might be helpful to show in class, since the dynamic modes are sometimes hard to imagine purely based on the equations given.

Regards,
Kyle

Continuation of the mathematical development related to (7):

With very little further effort, we can say even more. To this end, it must be noted that the components of both the left and right eigenvectors,  qktrand pk, respectively, are in general complex-valued. Hence, they may be expressed in polar form as:

pk=[p1k p2k ⋯pnk]tr =Δ [|p1k|eiθ1k |p2k|eiθ2k ⋯|pnk|eiθnk]tr 	(8a)

and

qktr=[q1k q2k ⋯qnk] =Δ [|q1k|eiϕ1k |q2k|eiϕ2k ⋯|qnk|eiϕnk].	(8b).

From (8b), we have:

αk=qktrx0= [|q1k|eiϕ1k |q2k|eiϕ2k ⋯|qnk|eiϕnk]] [x10 x20⋯xn0]tr = j=1n|qjk|eiϕjkxj0.	(9a)

The expression (9a) can be expressed simply as:

αk =Δ |αk| eiγk.	(9b)

From (8a) and (9b), the mth component of the response (7) becomes:

xm(t) = k=1n|αk||pmk|eskt+i(γmk+θmk) .	(10a)

In particular, the mth component of the kth mode, x(k)(t)=αkesktpk, has the form:

xm(k)(t) = |αk||pmk|eskt+i(γmk+θmk).	(10b)

From (10) we have a second answer to the question:

(A2): The magnitude of the mth component of the kth mode is controlled by the left and right eigenvectors, qktr and pkin the following manner:

(i) The magnitude of the mth component of the kth right eigenvector, pk, contributes directly, whereas

(ii) All of the component magnitudes of the left eigenvector, qktr, contribute indirectly, via x0.

Similarly, the phase of the mth component of the kth mode is controlled by the left and right eigenvectors, qktr and pkin the following manner:

(i) The phase of the mth component of the kth right eigenvector, pk, contributes directly, where as

(ii) All of the component phases of the left eigenvector, qktr, contribute indirectly, via x0.

3. Excitation of a Single Mode

An important question in relation to the system (1) is:

Exactly how does the choice of the initial condition, x0, influence the extent to which various modes are excited?

To answer this question, we use the following fact.

Fact 2: Define the matrix of right eigenvectors P=Δ [p1 p2 ⋯pn] , and the matrix of left eigenvectors

Q=Δ q1trq2tr⋮qntr. Then Q P = I, where I is the n×n identity matrix.

This fact is an extension of item (ii) of Fact 1, which states that qktrpk=1. Fact 2 goes further, stating that qjtrpk=0 for j≠k. In matrix terminology, the matrix Q is the left inverse of the matrix P. Hence, we have an answer to the above question:

A single mode, say the kth mode will be the only mode to be excited by the initial condition x0if and only if x0=c0pkfor some (possibly complex-valued) constant c0≠0.

Remark 1. The above answer requires a little elaboration. Specifically, suppose A includes a pair of complex conjugate eigenvalues, say sk=σk+iωk and sk+1=σk−iωk. It can be shown that in this case, pk+1=pk and qk+1tr=qktr. Since, in general, the components of pkare complex numbers, and since the components of x0are real numbers, then it is not possible for x0to excite only the kth mode. It must excite both the kth and kth+1modes. This requires that x0=c0(pk+pk+1). In simpler language, it is impossible to excite a real-valued oscillatory response without exciting both of the conjugate modes.

4. The Response Associated with a Real and with a Pair of Complex-Conjugate Eigenvalues

It turns out that an nth order system transfer function can be expressed as a sum of first order systems and second order underdamped systems. In this section we will focus on these two types of systems.

Case 1: The kth mode has sk = σk: In this case, (10b) becomes:

xm(k)(t) = |αk||pmk|eσkt+i(γmk+θmk) = (|αk||pmk|eσkt) ei(γmk+θmk).	(11)

Now, suppose that x0=c0pk, so that only this single mode is excited. Since (11) must be real-valued, it follows that we must have γmk=−θmk. But from (9a), we now have:

αk=qktrx0=c0qktrpk=c0

Recall that we require that x0=c0pkbe real-valued. Since the mth component of pkcan be expressed as pmk=|pmk|eiθmk and since we can express c0as c0=|c0|eiβ, it follows that the requirement that x0=c0pk be real-valued is equivalent to the requirements:

θ1k=θ2k=…=θnk=−β.	(12)

In general, the n requirements given in (12) will not be met exactly. The following special case is a not unrealistic situation where they will be met approximately.

Special Case- Suppose that pkis dominated by the mth component, pmk; that is, |pmk|>>|pjk| for all j≠m. For convenience, assume that we have scaled pk so that pk2=Δk=1n|pmk|2=1. Then

pk=[p1k p2k ⋯pnk]tr ≃ [0⋯0 eiθmk0 ⋯0]tr , 	(13a)

and the n requirements in (12) reduce to the single requirement θmk=−β. It follows that for c0=e−iθmk, the initial condition x0=c0pk≃[0 0⋯0 1m 0⋯0]trwill excite only the mth mode. If we assume that (13a) is an exact equality, then the condition qktrpk=1requires that

qktr=[q1k q2k ⋯qnk] ≃ [0⋯0 e−iθmk0 ⋯0].	(13b)

And so αk=Δqktrx0=e−iθmk. Then we have x(t)≃αkesktpk =[0⋯0 eσkt0⋯0]tr . Notice that throughout this special case we have used approximate equalities. That’s because rarely, if ever, will (13a) hold exactly.  □

Aircraft Response Due To Atmospheric Inputs (c.f. Nelson Chapter 6  p.212)

We begin this section by repeating the longitudinal and lateral small disturbance dynamical system equations, but with simplifications obtained by neglecting variables that have been described as negligible.

Neglecting Mw, the longitudinal state equation on p.4.12 of these notes is:

ΔuΔwΔqΔθ=XuXw0−gZuZwu00MuMwMq00010 ΔuΔwΔqΔθ +XδZδMδ0 XδTZδTMδT0ΔδΔδT.	(1)

By setting the product of inertia Ixz=0, the lateral equations on p.4.44 of these notes are:

ΔvΔpΔrΔφ = YvYpYr−u0gcosθ0LvLpLr0NvNpNr00100ΔvΔpΔrΔφ + 0LδaNδa0 YδrLδrNδr0ΔδaΔδr.	(2)

The atmosphere is not a uniform medium. It includes spatial and temporal dynamics associated with wind. Recall that the forces and moments acting on a plane depend on its motion relative to the local atmosphere. To demonstrate how the above equations are modified due to wind, we can write the plane x-axis velocity disturbance in relation to the local x-axis gust velocity, ug, as:Δua=Δu−ug. Repeating this for all disturbance variables gives:

linear velocities:         Δua=Δu−ug    ;   Δva=Δv−vg   ;   Δwa=Δw−wg

rotational velocities:   Δpa=Δp−pg    ;   Δqa=Δq−qg   ;   Δra=Δr−rg.

The above state equations then become:

ΔuΔwΔqΔθ=XuXw0−gZuZwu00MuMwMq00010 ΔuΔwΔqΔθ +XδZδMδ0 XδTZδTMδT0ΔδΔδT + −Xu−Xw0−Zu−Zw0−Mu 0−Mw 0−Mq 0 ugwgqg	(3)

ΔvΔpΔrΔφ=YvYpYr−u0gLvLpLr0NvNpNr00100ΔvΔpΔrΔφ +0LδaNδa0 YδrLδrNδr0ΔδaΔδr+−Yv00−Lv−Lp−Lr−Nv 0−Np 0−Nr 0 vgpgrg.	(4)

Both (3) and (4) have the structure of the following dynamical system:

x = Ax + B1u1 + B2u2.

In previous sections we ignored the control inputs. We will do the same here. In this case, the above form becomes:

x = Ax + Bu.	(5)

In both (3) and (4) the input, u, is a 3-D vector, and the output is a 4-D vector. Hence, in the jargon of modern control theory, these are said to be examples of a Multi-Input/Multi-Output (MIMO) system.

For a MIMO(3,4) system, each of the 3 inputs relates to each of the 4 outputs. Hence, there are a total of 12 scalar-valued relationships. The transfer function associated with (5) is obtained by taking its Laplace transform under zero initial conditions. Doing this, we obtain sX(s) = AX(s) + BU(s), or:

X(s) = (sI−A)−1 BU(s) =Δ H(s)U(s).	(6)

The matrix  (sI−A)−1B =Δ H(s) defined in (6) is the transfer function matrix that relates the input, u, to the output x.

In-Class Question: What is the dimension of H(s)?   Answer: 4x3

The (j,k) element of H(s)is a scalar-values transfer function that relates the jth output to the kth input. For example, in (1) the transfer function between the input Δδ(t) and the output Δu(t)is the (1,1) element of the transfer function matrix.

We are now in a position to compute the transfer function matrices associated with (1) and with (2) . We can do this simply by identifying the specific matrices associated with the matrices A and B in  (sI−A)−1B =H(s). To obtain the 12 scalar-valued transfer functions for each system by hand would be a very laborious task. It is conceivable that a symbolic code (e.g. Mathematica) could perform this task and yield nice expressions. We will not undertake either task at this time. The motivated student is encouraged to ‘go for it’!

A Closer Look At Scalar-Valued Transfer Functions

We have already seen the value of having a scalar-valued transfer function. It allows us to specify an input, u, and obtain the output, x, using very simple mathematics, a table of Laplace transforms, and Matlab commands. To motivate the discussion, we will consider the “PURE VERTICAL OR PLUNGING MOTION” of a plane, considered in Nelson, p.218. In this section a plane constrained to undergo only vertical motion in response to a gust of wind is addressed. The development begins with:

Forces in z−direction = m dwdt =Z+W.

Expressing the aerodynamic force in the z-direction as Z=Z0+ΔZ(i.e. a small perturbation force), results in Z0=−W(i.e. equilibrium), and writing w as w=ΔΔw(i.e. unaccelerated flight) gives:

ΔZm = dΔwdt.

Since the vertical force is a function of the angle of attack, Δα, and its derivative, Δα, this can be expressed as:

ZαΔα + ZαΔα − Δw= 0.	(1)

The change in the angle of attack due to the combination of its motion and the vertical wind gust is:

Δα≃Δwu0 − wgu0.	(2a)

This gives:                                         Δα ≃Δwu0−wgu0.	(2b)

(1−Zαu0)Δw−(Zαu0)Δw= −(Zαu0)wg+(Zαu0)wg.	(3)

As mentioned before, one standard for of a first order model has the form  τy(t)+y(t)=gsf(t), where the ‘input’ is f(t), the ‘output’ is y(t), the system time constant is τ, and the system static gain is gs. putting (3) in this form gives:

−u0−ZαZαΔw+Δw= (ZαZα)wg+wg.	(4)

Hence, the system time constant is  τ=−u0+ZαZα and the static gain is gs=1. The system transfer function is obtained by taking the Laplace transform of (4) assuming zero initial conditions. It is:

H(s)=ΔΔW(s)Wg(s)=(ZαZα)s+1−u0+ZαZαs+1 = (ZαZα)s+1τs+1 .	(5)

From (5), we can easily obtain the response for a variety of gust profiles. We now consider some progressively realistic profiles.

Case 1: A very idealized (and mathematically simple) gust profile.

Suppose that the gust profile, wg(t) has the form of a short burst of duration, T, and strength, Wo. To obtain its Laplace transform, Wg(s), we need three items from a table of Laplace transforms:

Item 1: The unit step function, us(t) has the form us(t)=1 for t>0. Its Laplace transform is Us(s)=1s.

Item 2: For any function f(t) with Laplace transform F(s), the time-shifted function r(t)=Δf(t−to) has Laplace transform R(s)=F(s) e−sto.

Item 3: For any function f(t) with Laplace transform F(s), the scaled function r(t)=Δcf(t) has Laplace transform R(s)=cF(s) .

The idealized gust pulse can be written as: wg(t)=Wo[us(t)−us(t−T)]. Using the above items, its Laplace transform is:

Wg(s)=Wo1−e−sTs.	(6)

From (5) and (6), the Laplace transform of the responseΔw(t) is:

ΔW(s)=H(s)Wg(s)= (ZαZα)s+1τs+1 1−e−sTs.	(7)

We can use a table of transform pairs to obtain the expression for Δw(t). But it is instructive to express (7) as the sum of two components:

ΔW(s)=H(s)Wo 1−e−sTs = H(s)Wo 1s − H(s)Wo 1se−sT. 	(8)

The first term on the right side of (8) is simply the scaled step response of the system H(s). The second term on the right is the same response, but time-delayed by an amount T.

EXAMPLE PROBLEM 6.1 of Nelson (p.224) As above, both Nelson and Etkin typically assume that Zα≃0. Then we have  τ=−u0Zα. Recall that Zα=−CLαQSm. Hence, τ=2W(CLαSρu0g). For a general aviation plane flying at 125 fps, we obtain τ≃0.7sec With these numerical values, the system transfer function is:

H(s)=ΔΔW(s)Wg(s) = 10.7s+1 =1.431s+1.43.

From entry #4 in the table of Laplace transform pairs, the corresponding system impulse response is:

h(t)=1.43e−1.43t.

We will assume that the gust magnitude is Wo=15 ft/sec, and that its duration is T=2 sec. . From entry #7 of the table and equation (8) above, the system response to this gust is:

Δw(t)=15 (1−1.43e−1.43t) − (1− 1.43e−1.43(t−2))us(t−2).	(9)

Note that in (9) we cannot cancel the two 1’s, since the second 1 is multiplied by the time-delayed unit step function. Consequently for any time t<2sec, (9) is only:

Δw(t)=15 (1−1.43e−1.43t) for 0<t<2 sec.	(10)

To plot the response (() using Matlab we have two options:

Option 1: Define an array of discrete time values that extend from 0 to at least (2+4τ) sec. We will choose the maximum time to be 5 seconds. The interval, ΔT, between samples should be small relative to τ. We will choose ΔT=0.01 sec. Hence the array of computation times will include a total of 1+5/.01=501 numbers. This array is computed as: tvec = 0:500;  tvec = tvec*.01. The response portion given by (10) is then simply: w = 15*(1 – exp(-1.43*tvec). Its time-delayed version begins after the 2/.01=200th time index. And so: ws = [zeroes(1,200) w]; ws = ws(1:501).

These commands are summarized as:

>> tvec=0:500; tvec=.01*tvec;

>> w=15*(1-exp(-1.43*tvec));

>> ws=[zeros(1,200) w]; ws=ws(1:501);

>> dw=w - ws;

>> plot(tvec,dw)

Figure 1. Plane vertical speed response to a 2-second idealized gust with magnitude 15 ft/sec.

FREQUENCY CONTENT ASSOCIATED WITH INPUTS, OUTPUTS & TRANSFER FUNCTIONS

The profile in Case 1 was mathematically convenient, and does provide some basic insight as to how a gust might affect the dynamics of an aircraft. But anyone who has stood in an open field on a windy day knows that gusts of wind, and the wind, in general, are more complicated.

If you watch a tree during a gust, you will find that it doesn’t simply bend in a static way. Rather, it sways. That’s because a tree is an underdamped system with a natural frequency. If you give it an initial condition by pulling it with a rope, and then let go, it will sway back and forth at its damped natural frequency. Now, the only way that a natural frequency can be excited is if the input has energy at that frequency. And so, returning to models for a wind gust, if the chosen model has no energy at the natural frequency of the plane dynamics, then the response to that gust model will not involve those dynamics. This could result in very misleading predictions to the plane’s response to a real gust.

The key word that was used repeatedly here is frequency. To this point, we have been concerned with the relation between a transfer function and time domain behavior (e.g. settling time and period of oscillation). And so, a natural question is:

QUESTION: How does a system transfer function, say, H(s), convey frequency information about the system dynamics?

To begin to answer this question, we will now prove a most fundamentally important connection between the time-domain (t) and Laplace-domain (s) representations of a system with transfer function H(s). To emphasize this relationship, we will not state it as a fact. Rather, we will state it as a theorem. The proof of this theorem is central to understanding it.

THEOREM. A system transfer function, say, H(s), is mathematically identical to the Laplace transform of the system impulse response.

Before proving this theorem, let’s be clear about the difference between a transfer function, H(s), and the Laplace domain expression for the system response to an input, say, F(s). Suppose, for example, that the input is a force, with units of lbf and that the response is displacement, with units of ft. Then the units of H(s) are [ft / lbf], while the units of the response, Y(s)=H(s)F(s)are ft. And so to say that the Laplace transform of the response to a unit impulse, δ(t) is equal to H(s)makes no sense. They have different units! The above theorem does not say they are equal. T says they have the same mathematical expression. We will use the force/displacement setting in the proof.

PROOF: For a unit impulse input, f(t)=δ(t), the Laplace transform is F(s)=1 [lbf]. The Laplace transform of the response, y(t), to any input, f(t), is simply Y(s)=H(s)F(s) [ft]. Hence, for the unit impulse input, this becomes Y(s)=H(s) [ft]. Recall, that the standard convention is to use a lower case for the time domain expression, and the upper case for its Laplace transform. And so, since Y(s)=H(s) [ft] we have y(t)=h(t) [ft]. The time-domain function h(t)is the inverse Laplace transform of the system transfer function, H(s). It is also the time-domain response to a unit-impulse input. Hence, h(t)is called the system impulse response function. In summary then, the system transfer function is the Laplace transform of the system impulse response function. □

EXAMPLE PROBLEM 6.1 continued: The impulse response of the system with transfer function H(s)=ΔΔW(s)Wg(s) = 10.7s+1 =1.431s+1.43 is h(t)=1.43e−1.43t.

This response is plotted against the response to a 2-second wind gust below.

Figure 2. Plane vertical speed response to a 2-second idealized gust with magnitude 15 ft/sec, and to an impulse with intensity equal to 30.

Discussion: The response to the 2-second pulse is well-behaved at t=0, whereas the impulse response makes a very sharp jump. This sharp jump means that the frequency content of the impulse response will include much higher frequencies that the frequency content in the 2-second pulse. But this may not be at all obvious to those who are new to this topic. To quantify this, we now give the answer to the above QUESTION:

ANSWER: The frequency content associated with a system with transfer function H(s)is obtained by setting s=iω. The quantity H(iω)is called the system Frequency Response Function (FRF).

It follows that the frequency content associated with the response y(t), to any input, f(t), is simply Y(iω)=H(iω)F(iω). For completeness, we give the following definition.

Definition 1. Let f(t) be a function of time, and assume that f(t)=0 for t<0. The function

F(s)=t=0∞f(t)e−stdtis called the Laplace Transform of f(t). For s=iω, this transform becomes

F(iω)=t=0∞f(t)e−iωtdt, and is called the Fourier Transform of f(t).

We are now in a position to quantitatively describe the frequency content associated with the two responses in Figure 2 above.

The frequency content associated with the response to the 2-second gust is obtained directly from (7) (in the context of EXAMPLE PROBLEM 6.1) as:

ΔW2sec(iω)=H(iω)Wg−2sec(iω)= 1.431.43+iω 1−e−i2ωiω,	(10a)

whereas, the frequency content associated with the impulse gust (with intensity = 30) is simply:

ΔWimpulse(iω)=H(iω)Wg−impulse(iω)= 1.431.43+iω 30.	(10b)

The frequency content associated with (10a) and (10b) is shown below.

Figure 3. The frequency content associated with (10a) BLUE and (10b) GREEN. NOTE: (10b) is the system (scaled) FRF.

Discussion: The frequency content of the two inputs is similar up to about 0.1 Hz. At higher frequencies the frequency content of the response to the 2-second pulse drops significantly relative to that associated with the impulse.

THE FREQUENCY CONTENT IN A RANDOM WIND PROFILE

Again, imagine you are standing in a field during gusty wind conditions. Clearly, no two gusts will feel exactly the same. In fact, the wind, in general is not exactly the same from second to second. And so, a more realistic model for a gust would be one that has elements of randomness to it. At issue here is not the energy content in a gust. Every gust will have different energy content. What is of central concern now is the expected frequency content. More specifically, it is the expected energy at each frequency.

Consider a function f(t). Its frequency description is given by its Fourier Transform:

F(iω)=0∞f(t)e−iωtdt. This is well-defined if f(t)decays fast enough as t→∞. (e.g. a decaying exponential.) But wind (esp. upper atmospheric wind) is always present. It does not simply go away after some 4τperiod of time. Furthermore, it has randomness. In fact, wind is an example of a random process. In this section we will assume it is a random process that has the same general behavior at in time. We will refer to such a process as a (weakly) stationary random process.

Definition 2.  Assume that f(t)is a stationary random process. Then the Fourier Transform of f(t) is defined as F(iω)=ΔlimT→∞1T0Tf(t)e−iωtdt for 0≤ω<∞.

Remark: The Fourier Transform operation converts a time-domain random function, f(t) ; 0≤t<∞, to a random function F(iω) ; 0≤ω<∞. It is simply a different way of viewing the random function (i.e. as a function of ωinstead of as a function of t). It does not reduce the level of variability.

Since F(iω) ; 0≤ω<∞is a random function, we need to address its frequency content in relation to the expected energy at any frequency. This is defined as:

Definition 3. The expected energy at a frequency ω is defined as E[|F(iω)|2]. In Nelson (p.227-228) the symbol used for it is E[|F(iω)|2]=ΔΦf(ω). In words, Φf(ω)is called the power spectral density (psd) function associated with the random process f(t). The symbol E( ) denotes the expected value operation.

Examples of Wind PSD Models in Relation to the (u,v,w) directions:

For a specified plane speed, u0, define the scaled frequency Ω=Δωu0. The following are the von Karman psd models for the three directions (u,v,w):

Φug(Ω)=σug22Lugπ1[1+(1.339LugΩ2]56	(11a)

Φvg(Ω)=σvg22Lvgπ1[1+(1.339LvgΩ2]116	(11b)

Φwg(Ω)=σwg22Lwgπ1[1+(1.339LwgΩ2]116	(11c)

The model parameter σ2 controls the overall intensity of the psd for a given direction, and the parameter L controls the turbulence scale in a given direction. These parameters are specified by the researcher.

EXAMPLE 6.1 continued: Since we are concerned in this example with vertical gusts, we will use equation (11c), but will express it as a function of ω=Ωu0:

Φwg(ω)= σwg22Lwgπ1+1.339Lwgu02ω2116

Connection to Matlab Simulink & to Military Specifications

# Von Karman Wind Turbulence Model (Continuous): Generate continuous wind turbulence with Von Kármán velocity spectra

## Library: Environment/Wind

## Description: The Von Kármán Wind Turbulence Model (Continuous) block uses the Von Kármán spectral representation to add turbulence to the aerospace model by passing band-limited white noise through appropriate forming filters. This block implements the mathematical representation in the Military Specification MIL-F-8785C and Military Handbook MIL-HDBK-1797.

According to the military references, turbulence is a stochastic process defined by velocity spectra. For an aircraft flying at a speed V through a frozen turbulence field with a spatial frequency of Ω radians per meter, the circular frequency ω is calculated by multiplying V by Ω . The following table displays the component spectra functions: [See Matlab documentation] □

Major Points re: AerE355:

1. In the real world, wind velocity is not modeled as a simple deterministic function.

2. Military specs. require that wind velocity be modeled by passing band-limited white noise through appropriate forming filters. .

Q1: What in the world is band-limited white noise?

A1: To answer this, we first need to understand what white noise is. A random process, call it w(t) , is a white noise process if its psd is: Φu(ω)=σu2 (i.e. it is a flat psd.) It follows that band-limited white noise is white noise that has been passed through a low-pass filter.

Q2: What is a low-pass filter?

A2: It is a transfer function whose FRF magnitude is unity at low frequencies and decays to zero at higher frequencies. The following first order system is an example of a low-pass filter.

Example of a low-pass filter- Consider the first order system with transfer function H(s)=gsτs+1. Recall, that the parameter τis the system time constant, and that gsis the system static gain. The system Frequency Response Function (FRF) is:

H(iω)=gsτ(iω)+1	(12a)

Since (12a) is a complex number, we can express it in polar form. [Recall that the polar form of the complex number a+ibis a2+b2 eiθ where θ=tan−1(ba) if both a,b≥0.] Hence,

H(iω)=gsτ(iω)+1=eiθ(ω)=M(ω)eiθ(ω)where M(ω)=gs1+(τω)2 and θ(ω)=−tan−1(τω)	(12b)

It is standard practice to express the magnitude, M(ω),  of the FRF in units called decibels (dB). Specifically: MdB(ω)=Δ20 log10[M(ω)]. Hence, in relation to (12), we have:

MdB(ω)=20log10gs1+(τω)2 = 20log10(gs) −10log10[1+(τω)2].	(13)

Things to note:

(i) at frequencies ω<<1τ, MdB(ω)≃20log10(gs) .

(ii) at frequency ω=1τ, MdB(ω)=20log10(gs) −10log10(2) =20log10(gs) −3dB

(iii) at a frequency ω1>>1τ, MdB(ω1)≃20log10(gs) −20log10(τω1)

at a frequency ω2=10ω1, MdB(ω2)≃20log10(gs) −20log10(τω2)  =20log10(gs) −20log10(τ10ω1)  =20log10(gs) −20log10(τω1) −20log10(10) =MdB(ω1) −20dB

In words, the FRF is flat at frequencies much lower than the frequency 1τ, and drops at a rate of 20dB per decade increase at frequencies much higher than the frequency 1τ. Finally, at the frequency 1τ, the magnitude has dropped 3dB relative to its low frequency magnitude. The frequency 1τ is called the system break frequency, since it is at this frequency that the zero-slope low frequency behavior begins to transition to the high frequency slope of 20dB/decade.

Now, to make this first order system function as a low-pass filter it is only necessary to set gs=1. Acting as a filter, the break frequency is called the filter cut-off frequency. Suppose that we desire a cut-off frequency fco=10 Hz.Then ωco=20π rad/sec, and the filter time constant is τ=1ωco=120π =0.016sec. For this filter: H(s)=1.016s+1 we can plot the FRF using two Matlab commands. First, we define the system: ‘sys = tf(1,[.016 1])’. Then we type ‘bode(sys)’.

The result is given below.

Figure 4. First order low-pass filter FRF with cut-off frequency fco=10 Hz.(i.e. ωco≃62.8 rs)

Application to the Von Karman Wind Model

We are now in a position to address the Von Karman wind model. For simplicity, we will express the vertical velocity psd as:

Φwg(ω)= σwg22Lwgπ1+1.339Lwgu02ω2116 Δ gs21+(τω)22

Notice that we can write 1+(τω)2=[1+τ(iω)] [1−τ(iω)] =[1+τ(iω)] [1+τ(iω)]_____________, where the bar denotes the complex conjugate. If we let s=iω, we then have

Φwg(ω)= σu21+(τω)22 = 1(τs+1)2 1(τs+1)2__________σu2 =ΔH(s) H(s)_____ H(s)2σu2

where we have defined the “transfer function

H(s) =Δ 1(τs+1)2

For any time, t, the white noise variance is σu2=E[u(t)2], where E(∗)denotes the expected value. Recall from Definition 3 above that E[|W(iω)|2]=ΔΦw(ω). Now, write

W(s)=H(s)U(s).

In words, we are representing the wind, w(t), as the output of the system H(s), with “input” u(t).

We then have

Φw(ω)=E[|W(iω)|2]=E[|H(iω)U(iω)|2]= |H(iω)|2E[|U(iω)|2] = |H(iω)|2σu2

Conclusion: We can simulate  Von Karman wind data by running white noise with variance σu2 through the transfer function H(s).

# Table of Laplace and Z Transforms

(Please email me if you find an error) Using this table for Z Transforms with Discrete Indices
Shortened 2-page pdf of Laplace Transforms and Properties

| Entry # | Laplace Domain | Time Domain | Z Domain (t=kT) |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |
| 11 |  |  |  |
| 12 |  |  |  |
| 13 |  |  |  |
| 14 |  |  |  |
| 15 |  |  |  |
| 16 |  |  |  |