Kalman Filtering with Known Forcing Function

Consider the following system:

xk+1=Fkxk+Gkwk+dk+1	(1a)

zk=Hkxk+vk+rk	(1b)

where the state and measurement processes involve known input functions dk and rk , respectively.

Recall, that the Kalman filter utilizes the error

ek=zk−zk	(2a)

multiplied by the Kalman gain, K, to recover an estimate of the state, xk where, in the case of (1),

zk=Hkxk+rk.	(2b)

The current and one-step ahead state estimates are

xk=xk−+Kkek	(3a)

and

xk+1−=Fkxk+dk+1.	(3b)

Now, let’s see how these modified measurement (2b) and state (3b) predictor equations are incorporated into the Kalman filter algorithm.

Step 1: For k=0 choose a value for x0−and compute the prediction error variance:

P0−=E[(x0−x0−)(x0−x0−)tr].

If we choose x0−=d0, then we have P0−=E[x0x0tr]where x0=Δx0−d0

Step 2: For k=0: Compute the Kalman gain:  Kk=Pk−Hktr (HkPk−Hktr+R)−1.

Step 3: Compute the update estimate: xk=xk− + Kk[zk−(Hkxk−+rk)].

Step 3: Compute the update error variance: Pk=(I−KkHk)Pk−.

Step 4: Compute 1-step prediction and associated error variance:

xk+1−=Fkxk+dk+1   ;    Pk+1−=FkPkFktr+GkGktr

Step 5: Go to Step 2 and increment k by one.   □

Example: Prediction of the 1-D position and velocity of a flexible robot arm:

The fluid has a mean viscosity that is included as a part of the viscous damping coefficient c. However, the fluid also has turbulence that results in zero-mean fluctuating loading of the mass.

To develop the equation for the motion of m, construct a free body diagram:

forces=my=−c(y−d)−k(y−d)−f. Hence, my+cy+ky=cd+kd−f. Assume the system is underdamped, and  define ωn=km , ζ=c2km , ωd=ωn1−ζ2. Then we have

y+2ζωny+ωn2y=2ζωnd+ωn2d−(1m)f.	(1)

The system (1) is a 2-input / single output system. We will use the impulse-invariant method of obtaining the discrete-time version of (1). To this end, we will require the following transforms:

The transfer function between d(t) and y(t) is:

Yd(s)D(s)=H1(s)=2ζωns+ωn2s2+2ζωns+ωn2=2ζωns+ωn2s2+2ζωns+(ζωn)2+ωn2−(ζωn)2=2ζωns+ωn2(s+ζωn)2+ωd2.

H1(s)=2ζωn⋅s+(ωn2ζ)+ζωn−ζωn(s+ζωn)2+ωd2=2ζωns+ζωn(s+ζωn)2+ωd2+(ωn2ζ)−ζωn(s+ζωn)2+ωd2

H1(s)=2ζωns+ζωn(s+ζωn)2+ωd2+[(ωn2ζ)−ζωn](ωdωd)(s+ζωn)2+ωd2.

Define c=[(ωn2ζ)−ζωn]ωd. We then have

H1(s)=2ζωns+ζωn(s+ζωn)2+ωd2+c⋅ωd(s+ζωn)2+ωd2	(2a)

The discrete transfer function associate with (2a) is therefore

H1(z)=2ζωn1−[e−ζωnTcos(ωdT)]z−11−2[e−ζωnTcos(ωdT)]z−1+e−2ζωnTz−2+c⋅[e−ζωnTsin(ωdT)]z−11−2[e−ζωnTcos(ωdT)]z−1+e−2ζωnTz−2

H1(z)=2ζωn1+e−ζωnT[c⋅sin(ωdT)−cos(ωdT)]z−11−2[e−ζωnTcos(ωdT)]z−1+e−2ζωnTz−2.	(2b)

The transfer function between f(t) and y(t) is:

Yf(s)F(s)=H2(s)=(1mωd)⋅ωd(s+ζωn)2+ωd2.	(3a)

The discrete transfer function associate with (3a) is therefore

H2(z)=(1mωd)⋅[e−ζωnTsin(ωdT)]z−11−2[e−ζωnTcos(ωdT)]z−1+e−2ζωnTz−2.	(3b)

Hence, we have

Y(z)=Yd(z)+Yf(z)=H1(z)D(z)+H2(z)F(z).

Specifically,

[1−2[e−ζωnTcos(ωdT)]z−1+e−2ζωnTz−2]Y(z)=2ζωn	(4a)

Define the parameters:

[a1=2[e−ζωnTcos(ωdT)] ; a2=−e−2ζωnT ; b1=2ζωn ; b2=2ζωne−ζωnT[c⋅sin(ωdT)−cos(ωdT)] ; b3=−(1mωd)e−ζωnTsin(ωdT).

Then the difference equation corresponding to (4a) is:

yk=a1yk−1+a2yk−2+b1dk+b2dk−1+b3fk−1.	(4b)

We will now assume that the fluid force adheres to an AR(2) model, so that spectral peaking associated with vortex shedding, etc. may be incorporated.

fk=c1fk−1+c2fk−2+uk.	(5)

Finally, we will assume that the mass incorporates a position sensor, and that there is sensor measurement noise. Then the measurement model is

zk=yk+vk.	(6)

We are now in a position to formulate the state and measurement equations associated with the Kalman filter parameters. To this end, define the known deterministic input∂k=b1dk+b2dk−1. Then (4b) becomes

yk=a1yk−1+a2yk−2+∂k+b3fk−1.	(7)

Now, define the state vector xk=[yk yk−1 fk fk−1]tr. From (5) and (7) we have

.	(8a)

zk=yk+vk=1000xk+vk ⇒ zk=Hxk+vk.	(8b)

From (8a) we have Q = diag{0 0 σu2 0}, and from (8b) R=σv2. For the state initial condition

x0−=[y0 y−1 f0 f−1]'+[∂0 0 0 0]'=[0 0 0 0]'+[∂0 0 0 0]'=[∂0 0 0 0]', the initial prediction error covariance is

.	(9)

Since is deterministic, it does not influence any covariances. Hence, (9) becomes:

(10)

Now recall that the state equation 8(a) for k=0:

(12)

Then, This gives:

(13a)

(13b)

Now:

.

Also:

Hence:                                                   	(14a)

(14b)

Substituting (14b) into (14a):               .	(15)

Equation (15) can be written as: . This is called the discrete-time Lyapunov Equation for the variable. The Matlab code ‘dlyap(F,Cu) will solve it.

As simple as the Matlab solution is, it lacks insight into the nature of the elements of . From (10) it is clear that those elements are auto- and cross-correlations at lags zero and one. So, one might ask: How can we arrive at these correlations via psd information. We now present a method that recovers all auto/cross-correlations based on psds.

Recall: and fk=c1fk−1+c2fk−2+ukgives

Hence, . From the Wiener-Kinchine Theorem we therefore have:

(i):

(ii):

(iii):

For numerical values, we can compute the quantities and then recover the associated correlation functions. Note that this is not necessary in relation to (i), as we have a direct method of solving for the autocorrelation for an AR(2) process.

NOTE: To recover the coefficient of a transfer function H, type: [num,den]=tfdata(H,’v’).

The code below recovers correlation functions from psds specified via transfer functions.

%PROGRAM NAME: psd2xcorr.m

%This code recovers Rxy from Sxy in dicrete time domain

%============================================

%Examples:

m=100;

%TF1:

H1=tf([1 0],[1,-.9],1); %Transfer Function

[H1n,H1d]=tfdata(H,'v');

H1nw=fft(H1n,m); H1dw=fft(H1d,m);

H1w=H1nw./H1dw;

%TF2:

%H2=H1; %For Autocorrelation

H2=tf([1 0],[1,-.4],1); %For Crosscorrelation

[H2n,H2d]=tfdata(H2,'v');

H2nw=fft(H2n,m); H2dw=fft(H2d,m);

H2w=H2nw./H2dw;

%Cross-Spectrum % Cross-Correlation

S12=H1w.*conj(H2w);

R12=real(ifft(S12));

figure(1)

plot(R12,'*')

title('Crosscorrelation for Two Specified TFs')

grid

Comment: I checked to make sure that the imaginary parts of the ifft’s were zero. They were. 