Lecture 13         Extended Kalman filter: Tracking a Time-Varying Sinusoid

2. The Time-Varying Sinusoid-Plus-Noise Model and Related State Equations

Consider what one might call a ‘real-world’ sinusoid:

s(t)=A(t) sin[θ(t)].	(1)

In this work, both the amplitude and frequency are subject to small amounts of random jitter about known (deterministic) amplitude, A0, and frequency, Ω0. The goal here is to use the extended Kalman filter (EKF) to estimate the time-varying amplitude and frequency. We assume here that (1) is corrupted by a second order Gauss-Markov noise process, n(t), so that the sampled measurement process is:

z(kΔ)=s(kΔ)+n(kΔ).	(2)

The following seven-dimensional state variable will be used:

x(t)=ω(t)Ω(t)θ(t)a(t)A(t)n(t)n(t−Δ)tr

where

- ω(t) is the frequency jitter (rad/sec)

- Ω(t)=ω(t)+Ω0 is the time-varying frequency (rad/sec)

- a(t) is the amplitude jitter

- A(t)=a(t)+A0 is the time-varying amplitude

- θ(t)=0tΩ(τ) dτ is the phase (rad) associated with the time-varying frequency

- n(t) is a random variable associated with a second order Gauss-Markov additive noise process n(t)

- n(t−Δ) is delayed an amount Δ from n(t).

2.1 The Sinusoid State Equations-

The frequency jitter model- The random frequency jitter, ω(t), will be modeled as:

ω(t)+βωω(t)=eω(t)	(3)

with power (i.e. variance) σω2, and with bandwidth parameter βω(rad/sec).  A common special case of (3) is when βω=0. In this case ω(t)is termed a random walk, or a Brownian motion. It is a model that is often used in relation to rate gyro position noise [*]. However, it is also a model that presumes that the uncertainty associated with ω(t) increases over time. In this work we will use βω>0, reflecting the assumption that the frequency jitter is centered about the nominal frequency Ω0, and has limited uncertainty, σωabout this value. It also reflects the assumption that the jitter has a limited bandwidth (in fact, a bandwidth that is small relative to Ω0). The model (3) considered in this work is called a first order Gauss-Markov (GM) model. It is intended to model slow frequency variations caused by road grade and head wind in relation to the use of automotive cruise control.

The amplitude jitter model- The amplitude jitter, a(t), is also assumed to be a first order Gauss-Markov process associated with the stochastic differential equation

a(t)+βaa(t)=ea(t)	(4)

with power σa2, and with bandwidth parameter βa(rad/sec) where βa>0 is also small relative to Ω0. Furthermore, in this work the processes (3 and (4) are assumed to be mutually independent. In a cruise control setting, this assumption may or may not be justified, depending on the source of the sinusoid.

The sampled processes- Because all processes are sampled using a sampling interval Δ (chosen so as to avoid measurable aliasing), the sampled versions of (3) and (4) (using impulse-invariant sampling) are, respectively,

ωk=αωωk−1+uk(ω) 	(5)

and

ak=αaak−1+uk(a).	(6)

In (5) and (6), and henceforth in this work, we will, for notational convenience, use the subscript notation for the time indices, as opposed to the notation in (2). The relation between the model parameters in (3-4) and (5-6) are, respectively,

αω=e−βωΔ     ;     σu(ω)2=σω21−(αω)2	(7)

and

αa=e−βaΔ     ;     σu(a)2=σω21−(αa)2.	(8)

As mentioned above, we assume here that both βω and βa are small relative to Ω0. We will assume that Ω0 is in the lower half of the frequency analysis range [0 , πΔ]. Hence, the bandwidth parameters αωand αawill both be close to 1.0. To quantify this closeness, we begin by noting that the sampled frequency jitter scaled autocorrelation function is

ρω(m)=(e−βΔ)m=αωm. Then, for a vehicle traveling at 70 miles per with an engine crankshaft frequency Ωo=2π(50) rads. (3000 RPM) we would need to have a sampling frequency fsamp≃5(50)=250 Hzin order to have Ω0positioned at 40% of the analysis bandwidth of 125 Hz. The vehicle speed of 70 mph corresponds to ~103 fps. Hence, if we assume road grade changes and head wind correlation length of ~100 ft, then only well after 1 second will the jitter process ω(t) have small ρω(m). If we assume that ρω(m1)≃0.1 for m1Δ≃1 sec. then m1=250samples.

In this case, we then have 0.1=αωm1=αω250, or αω≃0.99.

To obtain the difference equation for the phase, write:

θk=0kΔΩ(τ) dτ=0(k−1)ΔΩ(τ) dτ+(k−1)ΔkΔΩ(τ) dτ ≃ θk−1+ΩkΔ	(9)

where

where the rightmost approximate equality assumes that the sampling interval Δ is sufficiently small that for (k−1)Δ<τ<kΔ we have Ω(τ)≃Ω[(k−1)Δ].

The final two sinusoid discrete-time state equations are

Ωk=Ω0+ωkΔ	(10)

and

Ak=A0+ak.	(11)

Combining (5), (6), (9), (10) and (11) gives

xk =Δ ωkΩkθkakAk = α(ω)0000α(ω)00000Δ100000α(a)0000α(a)0 ωk−1Ωk−1θk−1ak−1Ak−1 + uk(ω)uk(ω)0uk(a)uk(a) + 0Ω000A0 =Δ F(s)xk−1(s) + uk−1(s)+dk(s).	(12)

2.2 The Noise State Equations-

As mentioned above, in this work we assume the additive noise corresponds to a second order Gauss-Markov process, with stochastic differential equation

n + β1n + β2n = en(t).	(12)

Furthermore, we will assume that the characteristic polynomial associated with (12) has the form

s2 + β1s + β2 = s2 + (2ζωn)s + ωn2	(13)

with complex roots. The parameter ζis the damping ratio, and the parameter ωnis the undamped natural frequency. The discrete-time version of (12) is then

nk+1=−b1nk−b2nk−1+ek+1(n)	(14)

where b1=−2 e−ζωnΔcos(ωdΔ)and b2=e−2ζωnΔ. Equation (14) can be written as:

xk(n) =Δ nknk−1= −b1−b210 nk−1nk−2+ ek(n)0 + 00 =Δ F(n)xk−1(n) + uk−1(n) + dk(n).	(15)

3. The Kalman Filter Equations

3.1 The Kalman Filter State Equations

The Kalman filter state equation is:

xk+1=F xk+uk+dk+1	(16)

where, from (12) and (15), we have xk =xk(s)xk(n) tr, uk =uk(s)uk(n) tr, dk =dk(s)dk(n) tr, and F=diagF(s)F(n). The state driving white noise, dk, has the following covariance matrix:

Q=diagQ(s)Q(n)	(17a)

where

Q(s)=σu(ω)2σu(ω)2000σu(ω)2σu(ω)200000000000σu(a)2σu(a)2000σu(a)2σu(a)2      and     Q(n)=σu(n)2000.	(17b)

For the chosen initial condition estimate x−1 =0W000A000tr,

the mean-squared error (mse) of the estimator, x0−=x−1 is simply

P0−=ΔE[(x0−x0−)(x0−x0−)tr]=E(x0x0tr)=σω2σω200000σω2σω20000000σθ020000000σa2σa200000σa2σa20000000Rn(0)Rn(1)00000Rn(1)Rn(0)	(18)

where σω2and σa2are the variances of the jitter processes ω(t) and a(t), respectively, and where σθ02=π23 assumes that θ0is uniformly distributed over the interval[−π,π).

3.2 The Kalman Filter Measurement Equations

In view of the results of the last section, the measurement equation (1) can be written as

zk = Aksin(θk) + nk = h(s)(xk(s)) + 010 xk(s)xk(n).	(19)

where  h(xk)=Aksin(θk). Because the Kalman filter is a linear filter, it is necessary to linearize h(xk). To this end, a first order Taylor series expansion about the sinusoid state estimate xk(s) −=ωk−Wk−θk−ak−Ak−tris used. Specifically,

h(s)(xk−) ≃ Ak−sin(θk−) + [Ak−cos(θk−)] (θk−θk−) + [sin(θk−)] (Ak−Ak−).	(20)

This, in turn, can be rewritten as

h(s)(xk−) ≃ [Ak−cos(θk−)] θk + [sin(θk−)] Ak − [Ak−cos(θk−)] θk− =Δ h3(s)θk + h5(s)Ak − dk.	(21)

From (21) it follows that (19) may be written as

zk = Aksin(θk) + nk ≃ 00h3(s)0h5(s)10 xk − dk =Δ Hkxk − dk.	(22)

where the matrix Hk=00h3(s)0h5(s)10 is used in the computation  of the Kalman filter gain

Kk=Pk−Hktr (HkPk−Hktr)−1.	(23)

It should be noted that the rightmost term in (22) is not used, since a more accurate estimate of the measurement zkis simply

zk = Ak−sin(θk−) + nk−.	(24)

The value of the exercise (20-22) is that it identified a structure for Hk, which is needed to compute Kkin (23).

4. Examples

The following examples relate to the time-varying sinusoid-plus process described in Table 1. The sampling interval is Δ=1 sec.

TABLE 1. Time-Varying Sinusoid and Colored Noise Parameter Values (Δ=1 sec.)

Sinusoid:

Frequency: F0=Ω02π=0.2Hz ; σf=σω2π=.005 Hz ; βf≃.16 Hz ; αω=0.996

Amplitude: A0=1.0 ; σa=.023 ; βa≃.16 ; αa=0.996

Noise:    σn=.707 ; fn=ωn2π=0.106 Hz ; ζ=0.1

From Table 1 we see that the 3 σvalue of both  the frequency and amplitude  jitter is approximately 7% of the nominal values. To arrive at the value for the frequency jitter parameter αω=0.996, first note that the nominal frequency F0=Ω02π=0.2Hz is a normalized frequency for an analysis range [0 , 0.5] Hz. Assuming that this normalized frequency corresponds to an engine frequency of 50 Hz, then the real time sampling frequency is 250 Hz, which corresponds to a sampling period Δ=.004 sec. To mimic the influence of changing road grade on the fluctuations in cruise control speed, it was assumed that the frequency jitter 4τωde-correlation time was approximately  4 seconds. Hence, the jitter real time bandwidth isβω≃1 rad/sec[or βf≃.16 Hz]. The result is that αω=e−βωΔ=0.996. The amplitude jitter was assumed to behave in the manner of the frequency jitter.

COMMENTS:

- The data generation and KF codes are included in the Appendix. The student is encouraged to run it.

- A more in-depth investigation of this problem is given in the Lectures folder. It is entitled: Extended Kalman Filtering- Tracking a Time-Varying Sinusoid. It is included, in part, to give the student ideas as to the type of investigation one might pursue in the context of a 573 project.

Partial Output:

Appendix   Matlab Code

% PROGRAM NAME: sinegen_AND_sinekfdelt2018.m

%PURPOSE: generate a realization of a t.v. sinusoid+noise

%=======================

npts=6000;   % LENGTH OF REALIZATION

ntot=npts + 500;

%===============================================================

% GENERATION OF TIME-VARYING SINUSOID with AR(1) Ampl. & Freq.

% ==============================================================

% NOMINAL AMPLITUDE AND FREQUENCY:

a0=1.0; %nominal amplitude

w0=2*pi*0.2; % nominal frequency

%======================

sea=0.01;      % amplitude driving noise sigma

sew=0.01;      % angular frequency driving noise sigma

sea2=sea^2;

sew2=sew^2;

%=======================

% Generation of white processes

eda=sea*randn(1,ntot);

edw=sew*randn(1,ntot);

% Compute variances of a(t) and w(t)for use in EKF

aa=0.9; aw=0.95;

sa2=sea2/(1-0.9^2);

sw2=sew2/(1-0.95^2);

%=====================

% Specify Initial Conditions

da(1)=eda(1);

dw(1)=edw(1);

theta(1)=0;

a(1)=a0;

w(1)=w0;

% Generate SIGNAL Realization

for t=2:ntot

da(t)=aa*da(t-1)+eda(t);

a(t)=a0 + da(t);

dw(t)=aw*dw(t-1)+edw(t);

w(t) = w0 + dw(t);

theta(t) = theta(t-1) + w(t);

end

%=====================

%Plot True Freq. and Ampl.

ftrue=w(501:ntot)/(2*pi);

atrue=a(501:ntot);

figure(1)                  % FIGURE 1

plot(ftrue)

title('Actual T.V. SINUSOID Frequency')

xlabel('Time [sec]')

ylabel('Frequency [Hz]')

figure(2)                 % FIGURE 2

plot(atrue)

title('Actual T.V.SINUSOID Amplitude')

xlabel('Time [sec]')

ylabel('Amplitude')

% Compute and plot T.V. Sinusoid Realization

s=a.*sin(theta);

s=s(501:ntot);

figure(3)                % FIGURE 3

plot(s)

title('T.V. Sinusoid s(t) with A=1 & w=0.2')

% =======================================================

%         ADDITIVE NOISE REALIZATION AND PSD

% =======================================================

% Generate AR(2) Noise Process

a1=-1.6*cos(pi/5); a2=0.64;

Rn0=1.0; %Noise variance

%Compute Rn1 and sn2:

AA=[-a1 -a2 1;-(1+a2) 0 0;-a1 -1 0]; BB=Rn0*[1;a1;a2];

RR=AA^-1*BB;

Rn1=RR(1); Rn2=RR(2);

su2=BB(3); % additive AR noise sigma of white noise input

su=su2^.5;

u=su*randn(1,ntot);

% Compute and Plot Noise PSD

delf=1/2048;

fvec=0:delf:0.5-delf;

den=fft([1 a1 a2],2048);

den=den(1:1024);

ARspec=su2*(abs(den)).^-2;

ARspecdB=10*log10(ARspec);

figure(4)                   % FIGURE 4

plot(fvec,ARspecdB)

ylabel('dB')

xlabel('Frequency (Hz)')

title('AR(2) Noise PSD')

nar=zeros(1,ntot);

nar(1)=u(1);

nar(2)=u(2);

for t=3:ntot

nar(t)=-a1*nar(t-1)-a2*nar(t-2)+u(t);

end

n=nar(501:ntot);

% =======================================================

%    CONSTRUCTION OF MEASUREMENT PROCESS REALIZATION

% =======================================================

z=s + n;

figure(5)                  % FIGURE 5

nvec=1:npts;

plot(nvec,z, 'b')

hold on

plot(nvec,s,'m')

hold off

title('T.V. Sinusoid with & without AR(2) Noise')

[1 a1 a2]

%****************************************************

%****************************************************

%****************************************************

%****************************************************

% PROGRAM NAME: sinekf_7D_ver3.m contained in 573 2013 folder

% PURPOSE: Track a t.v. sinusoid corresponding to sinegenver3.m

% with 7-D state

% REQUIRED INPUT:

%   z-array from program sinegen.m

%======================================

% Model Matrices for xk = [wk Wk thk ak Ak nk nk-1]

% for deterministic input d = [0 w0 0 0 a0 0 0]':

Phi = [ aw 0 0  0 0   0  0; % wkm1 <--wn sew2

aw 0 0  0 0   0  0; % Wkm1

0  1 1  0 0   0  0; % akm1 <--wn sea2

0  0 0 aa 0   0  0; % thkm1

0  0 0 aa 0   0  0; % Akm1

0  0 0  0 0 -a1 -a2;% nkm1  <--wn su2

0  0 0  0 0   1  0];% nkm2

R=0; %Measurement noise variance - ideally ZERO

%=====================================

% EKF Initial Conditions

xm = [0 w0 0 0 a0 0 0]';

x=xm;

Pm = [sw2     sw2    0     0    0     0   0;

sw2     sw2    0     0    0     0   0;

0       0     0     0    0     0   0;

0       0     0    sa2   sa2   0   0;

0       0     0    sa2   sa2   0   0;

0       0     0     0    0    Rn0 Rn1;

0       0     0     0    0    Rn1 Rn0];

%====================================

% EKF Loop

I = eye(7);

for k=1:npts

d = [0 w0 0 0 a0 0 0]';

Q = [sew2  sew2  0     0          0      0  0;

sew2  sew2  0     0          0      0  0;

0    0     0     0          0      0  0;

0    0     0   sf*sea2    sf*sea2  0  0;

0    0     0   sf*sea2    sf*sea2  0  0;

0    0     0     0          0     su2 0;

0    0     0     0          0      0  0];

H = [0 0 xm(5)*cos(xm(3))  0   sin(xm(3)) 1 0];

K = Pm*H'*(H*Pm*H' + R)^(-1);

zm = xm(5)*sin(xm(3))-a1*xm(6) - a2*xm(7);

xhatk=xm + K*(z(k) - zm);

x=[x,xhatk];

P=(I - K*H)*Pm;

xm=Phi*xhatk + d;

Pm=Phi*P*Phi' + Q;

end

x=x(:,2:npts+1);

tvec = 2:npts+1;

what = x(2,:);

fhat=(2*pi)^-1 * what;

thetahat = x(3,:);

ahat= x(5,:);

nhat = x(6,:);

%=========================================================================

tvec = 1:npts;

%==========================================================================

%    PLOTS OF STATE AND ESTIMATION ERROR PROCESSES

% STATE PLOTS:

figure(10)

%plot(tvec,f,tvec,fhat,'r');

subplot(4,1,1), plot(tvec,ftrue,tvec,fhat,'r');

legend('True','KF Estimate')

title('(a):   Sine T.V. Frequency(blue) and EKF Estimate(red)')

xlabel('Time [sec]')

ylabel('Frequency [Hz]')

%xlabel('Time (sec)')

grid

%figure(11)

%plot(tvec,atrue,tvec,ahat,'r');

subplot(4,1,2), plot(tvec,atrue,tvec,ahat,'r');

legend('True','KF Estimate')

title('(b):   Sine T.V. Amplitude(blue) and EKF Estimate(red)')

%xlabel('Time [sec]')

ylabel('Amplitude')

%xlabel('Time (sec)')

grid

%=====================

% Reconstruct Signal Estimate

shat=ahat.*sin(thetahat);

%figure(12)

%plot(tvec,s,tvec,shat,'r');

subplot(4,1,3), plot(tvec,s,tvec,shat,'r');

legend('True','KF Estimate')

title('(c):   T.V. Sine(blue) and EKF Estimate(red)')

%xlabel('Time (sec)')

grid

%figure(13)

%plot(tvec,n,tvec,nhat,'r');

subplot(4,1,4), plot(tvec,n,tvec,nhat,'r')

legend('True','KF Estimate')

title('(d):   AR(2) Noise (blue) and EKF Estimate (red)')

xlabel('Time (sec)')

grid

% ESTIMATION PERCENT ERROR PLOTS:

figure(11)

%plot(tvec,f,tvec,fhat,'r');

ef = 100*(fhat-ftrue)./ftrue;;

subplot(4,1,1), plot(tvec,ef);

title('(e):   Sine T.V. Frequency Percent Error')

xlabel('Time [sec]')

ylabel('%')

grid

ea = 100*(ahat-atrue)./atrue;

subplot(4,1,2), plot(tvec,ea);

title('(f):   Sine T.V. Amplitude Percent Error')

%xlabel('Time [sec]')

ylabel('%')

grid

% Reconstruct Signal Estimate

shat=ahat.*sin(thetahat);

rse_s=sqrt(mean(s.^2));

es = (100/rse_s)*(shat-s);

subplot(4,1,3), plot(tvec,es);

title('(g):   T.V. Sine Percent Error')

ylabel('%')

grid

rse_n=sqrt(mean(n.^2));

en = (100/rse_n)*(nhat-n);

subplot(4,1,4), plot(tvec,en)

title('(h): AR(2) Noise Percent Error')

ylabel('%')

xlabel('Time (sec)')

grid

%=======================

% Compute estimated percent mse's:

rmse_f=100*sqrt(mean((ftrue-fhat).^2)/mean(ftrue.^2));

rmse_a=100*sqrt(mean((atrue-ahat).^2)/mean(atrue.^2));

rmse_s=100*sqrt(mean((s-shat).^2)/mean(n.^2));

rmse_n=100*sqrt(mean((n-nhat).^2)/mean(n.^2));

[rmse_f rmse_a rmse_s rmse_n]