Kalman Filtering and Probability

1. A Brief Summary of Kalman Filtering in Relation to Random Variables

1.1 The State and Measurement Models

Let Zk=[Zk1,…,Zk m]trand let  Xk=[Xk1,…,Xk n]tr. Assume the relationship between these random processes is:

Xk+1=FkXk+Wk  (state equation)   ;  X0−(initial condition)	(1a)

Zk=HkXk+Vk (measurement equation)	(1b)

where (1a) includes the known and non-random (n×nmatrix-valued) parameters Fk and Gk, as well as the n-D non-measurable white noise random process wk with

E(WkWk+jtr)=Q⋅δ(j) and E(VkVk+jtr)=R⋅δ(j).	(2)

The Random Variables involved in the state model- There are two ways in which random variables can enter into (1a). The first way is via the process Wk=[Wk1,…,Wk n]tr. For any k, this n-D random variable need not have all non-zero entries. In fact, it often does not. Furthermore, the non-zero random variables may have distinctly different distributions. For example, one component might be normally distributed, while another is uniformly distributed, while a third has a Bernoulli distribution. Moreover, at any time, k, the elements of Wk can be correlated. The only requirement is that the possibly nonstationary random process,Wk,  be zero-mean and have mutually uncorrelated elements. The second way that randomness can enter into (1a) is via the initial condition; which may also entail zeros, as well as various types of random variables. It is possible that the state process (1a) has no driving noise input, and has a deterministic external input; but where the initial condition is a random variable. In this case, (1a) is an example of a deterministic random process, in that conditioned on knowledge of Xk for a given time, k, it is known for all future time.

Random Variables involved in the measurement model- Clearly, the randomness will arise, in part, from the state process. In addition to this, it may also have a contribution from the measurement noise kth element of the random processVk. This process has the same amount of allowed variety of random variables, and has the same constraint as the state noise process Wk.

1.2 The Kalman Filter Algorithm

From Figure 5.8 on p.219, we have the following KF algorithm:

For k=0 choose a value X0−=x0−and compute the prediction error covariance

P0−=E[(X0−X0−)(X0−X0−)tr]. 	(3)

[Notice that by choosing x0−=0, it follows that P0−=E[X0X0tr]. ]

Step 1: For k=0:      Compute the Kalman gain:  Kk=Pk−Hktr (HkPk−Hktr+Rk)−1.	(4a)

Step 2: Compute the update estimate:    Xk=Xk− + Kk(Zk−HkXk−).	(4b)

Step 3: Compute the update error variance:   Pk=(I−KkHk)Pk−.	(4c)

Step 4: Compute 1-step prediction:  Xk+1−=FkXk	(4d)

Step 5: Compute the next state error covariance: Pk+1−=FkPkFktr+Qk	(4e)

Step 6: Return to Step 1 and increment k by one.

Note that while steps 1-3 must precede steps 4 and 5, the order of the latter is irrelevant.

2. Probability Distributions Related to the State and Measurement Random Variables

2.1 Distributional knowledge related to Xk

Recall that the point of Kalman filtering is to recover an estimate of Xk from the measurements Zj≤k Hence, even though Xk is a random variable, we do not have access to it.

2.1.1 The mean and covariance of Xk|Xk− = xk−:

Recall that Xk− is the KF estimator of Xk prior to observing Zk. Let Ek−=ΔXk−Xk− denote the error associated with this estimator. This error has zero mean and covariance denoted as:

Pk−=E[(Xk−Xk−)(Xk−Xk−)tr] =Δ E(Εk−Εk− tr).	(5)

Suppose that we are given Xk− = xk−. Since Xk = Xk− + (Xk−Xk−)=ΔXk−+Ek−, it follows that

Xk |Xk− = xk− = xk− + Εk−.	(6a)

Cleary, then

E(Xk |Xk− = xk−) = xk−,        and           Cov(Xk |Xk− = xk−) = Cov(Ek−) = Pk−.	(6b)

Thus, while we do not know anything directly in relation to Xk, we do know the mean and covariance of the conditional random variableXk |Xk− = xk−, are as is given in (6).

2.1.2 The mean and covariance of Xk|Xk = xk:

Recall that Xk is the KF estimator of Xk that incorporates Zk. Let Ek=ΔXk−Xk denote the error associated with this estimator. This error has zero mean and covariance denoted as:

Pk=E[(Xk−Xk)(Xk−Xk)tr] =Δ E(ΕkΕktr).	(7)

Suppose that we are given Xk− = xk. Since Xk = Xk + (Xk−Xk)=ΔXk+Ek, it follows that

Xk |Xk = xk = xk + Εk.	(8a)

Cleary, then

E(Xk |Xk = xk) = xk,        and           Cov(Xk |Xk= xk) = Cov(Ek) = Pk.	(8b)

From Step 2 of the KF algorithm, we have

and                                 Xk=ΔXk−+Kk(Zk−Zk−) with  Zk−=ΔHkXk−.	(8c)

2.2 Distributional knowledge related to Zk|Xk−=xk−

Since Zk depends on Xk, via (1b), we do not have any a priori knowledge of the distribution of Zk. However,

we may write (1b) as

Zk=HkXk+Vk = Hk(Xk−+Εk−) + Vk.	(9)

Define Zk−=ΔHkXk−and zk−=ΔHkxk−. It follows immediately that

Zk|Xk−=xk− = Hkxk− + HkΕk−+Vk = zk− + HkΕk−+Vk.	(10a)

The mean and covariance of (10a) are given by

E(Zk|Xk−=xk−)=Hkxk− = zk−    and   Cov(Zk|Xk−=xk−)= + HkPk−Hktr+Rk.	(10b)

Finally, notice that the variable Zk|Xk=xk is not a random variable, but a number, zk. This is because to be given Xk=xk requires that we are given Zk=zk.

2.3 Application of Baye’s Theorem

In its simplest form, Baye’s Theorem may be stated as

Baye’s Theorem                Pr(A|B)=Pr(B|A) Pr(A)/Pr(B).	(11)

Proof: The proof is a trivial consequence of the definition of the conditional probability

Pr(A|B)=ΔPr(A∩B) /Pr(B). We may also write this as Pr(B|A)=ΔPr(B∩A) /Pr(A).

Since Pr(A∩B)=Pr(B∩A) , the result (11) follows immediately. □

Now, let Zk|Xk−=xk− =Δ W, and let A be a random variable with sample space SA. Then, from (11), we have, Notationally, Pr(A|W)=Pr(W|A) Pr(A)/Pr(W). However, whereas (11) relates to events A and B, this expression relates to random variables. Hence, more formally, (11) becomes

f(a|w)=f(w|a)f(a)f(w)	(12a)

where

f(w)=SAf(w,a) da	(12b)

Example. Consider the dynamical system

Xk=a Xk−1 + Uk	(13a)

Zk = Xk + Vk.	(13b)

Now, suppose that the number a ∈ a1 , a2=SA, with Pr[A=a1]=p. Then (12) becomes

Pr[A=a1|W=w]=f(w|a1)f(w,a1)+f(w,a2) Pr[A=a1].

This can also be written as

Pr[A=a1|W=w]=f(w|a1)f(w|a1)Pr[A=a1]+f(w|a2)Pr[A=a2] Pr[A=a1].	(14a)

From (10b) we have

E(W|a)= zk−    and   Cov(W|a)= + Pk−(a)+ σV2.	(14b)

Then for the event [W=w=zk−zk−], (14a) becomes

Pr[A=a1|zk−]=f(zk−zk−|a1)f(zk−zk−|a1)Pr[A=a1]+f(zk−zk−|a2)Pr[A=a2] Pr[A=a1].	(15a)

Equation (15a) may be viewed as an updated specification of Pr[A=a1], based on having zk. With this viewpoint, we express this updated probability as

Pr[A=a1] =Δ Pr[A=a1|zk−1−].	(15b)

Substituting (15b) into (15a) results in

Pr[A=a1|zk−]=f(zk−zk−|a1)f(zk−zk−|a1)Pr[A=a1|zk−1−]+f(zk−zk−|a2)Pr[A=a2|zk−1−] Pr[A=a1|zk−1−]	(16)

For convenience, assume that W|a ~ N(zk− , Pk−(a)+σV2).

We return to the first example of the semester.

Example. Consider

zk=akzk−1+vk ; E(vk2)=σv2(k)	(17a)

In the case where the AR(1) parameter ak changes slowly in relation to the sampling interval. In this sense, the process (16a) is a locally wss process, and it is easy to show that the process variance (or power) is given by

σz2(k)≃σv2(k)(1−ak2).	(17b)

The model (17) has the ability to capture the slow time variation in both the process power, and time-varying frequency content can be captured by the AR(1) parameter, ak.

Suppose that the AR(1) parameter is changing in the manner described in Figure 1.

Figure 1. Time-varying AR(1) parameter, ak.

Figure 2 shows a realization of this process. Note that the bandwidth is change, but the process power is not.

Figure 2. Measurement of zk associated with (3) and Figure 1. How was this generated?

Method 1: Here we will concentrate on tracking ak. In this case we will use measurements of the process (16) to filter out estimates of the ak’s. The state model used here is a simple random walk model:

ak=ak−1+wk ; Qk=σw2	(18)

Hence, the quantities in the KF state/measurement model (1) are:

xk=ak; Fk=Gk=1; Qk=σw2; Hk=zk−1;Rk=σv2(k).

There are two items here that make this KF suboptimal in the sense of minimizing the mean squared error between the state process xk and any estimator of it:

1.The parameter Hk=zk−1is not a non-random quantity. However, because at time k we have knowledge of zk-1, it is conditionally non-random. In this sense, the KF is known as an extended KF.

2.We do not have knowledge of Rk=σv2(k), which is the driving white noise for the process (17a).

However, we do know that the process power (17b) is constant and known. Hence, with our estimate akwe can estimate this white noise variance asσv2(k)=max0 , σz2(1−ak2) .

An example of the EKF tracking for two specified values of Qk=σw2 is shown in the plots below.

Figure 3. EKF estimates of ak for Qk=σw2: 0.00001 and 0.001.

The trade-off between bias and variability is clear. What is not clear is how this translates into change detection. If we define change as change in relation to the value 0.9, then Figure 3 would include numerous false detections for a threshold value of, say, 0.8. If we desire to detect whether ak is 0.9 or 0.4, with say a threshold value of 0.65, then the only incorrect detections occur in the transition region. The following method is designed to rapidly detect whether ak has one of two specified values.

Method 2: For this method, the ramp was replaced by an instantaneous change in the AR(1) parameter

Figure 4. The top plot shows the behavior of the AR parameter. The bottom plot shows a partial realization of the measurement process.

In this method we will not track ak. Instead, we will cast this as a problem of determining which of two known values of ak the measurements are associated with. In this setting, we will use the probability evolution model (16) in relation to each value. In this way, we are actually running TWO EKFs, and using (16) to decide which one is related to the true value of ak.

The results are shown in Figure 5.

% PROGRAM NAME: mmae411.m

% Most Recent Modifications 4/16/2011

% =================================

% Multiple Model Adaptive Estimation- an Example

% AR(1) Parameter Probability Information

% =================================

% DATA GENERATION

n = 10^4;

% Construct t.v. AR(1) parameter

a11=.9; a22=.4;

a1=a11*ones(1,n/2);

a2=a22*ones(1,n/2);

avec=[a11 ; a22]; % Vector of the two parameters being tested

a=[a1 a2];

figure(1)

plot(a)

axis([1,n,0,1])

title('Timevarying AR(1) Parameter')

pause

% Compute driving noise variance for unity variance AR(1)

sige2=(1 - a.^2);

% Generate t.v. AR(1) Measurement Process

x = zeros(1,n);

x(1)=randn(1,1);

for t=2:n

x(t)=a(t)*x(t-1) + sige2(t)^(0.5)*randn(1,1);

end

figure(2)

plot(x,'k')

title('Sample of unit variance t.v. AR(1) process')

pause

R=0.1;              %  MEASUREMENT NOISE VARIANCE

v=sqrt(R)*randn(1,n);% WHITE NOISE

z=x+v;               % MEASUREMENT

%=================================================================================

%   Use of TWO Simultaneous Kalman Filters to Estimate Pr(A1) and Pr(A2) = 1-Pr(A1)

%=================================================================================

% PROBABILITY INITIAL CONDITIONS

pr=[];

pra=[0.5;0.5]; % Assign equal prior probabilities to a1 & a2

pnm1=pra;      % pdf for z(0)=[0 0]

%========================================================

% KALMAN FILTER INITIAL CONDITIONS

Xhat=[];

Pmat=[];

Kmat = [];

ehatm=[];

Q=1-avec.^2;  % white noise variances for sigma_x=1

xhatm=[0;0];

Pm=[1;1];     % Note that R has been specified above

%=======================================================

for k=1:n

% COMPUTATION OF pr(k) = [pk;a1) p(k;a2)]'

zk=z(k)*[1;1];

p1coef=(2*pi*(Pm+R)).^(-0.5);

p1=p1coef.*exp((-0.5*(zk-xhatm).^2)./(Pm+R));

pd=p1'*pnm1;        %This is the sum of TWO probabilities

p=(1/pd)*p1.*pnm1;  % NOTE that pden is scalar-valued

[pmax,i]=max(p);

if pmax > .99

p(i)=.99;

j=1+mod(i,2);

p(j)=1-p(i);

end

pr(:,k)=p;

pnm1=p;

% KALMAN FILTER COMPUTATIONS

K=Pm.*(Pm+R).^(-1);

Kmat=[Kmat,K];

P=([1;1]-K).*Pm;

Pmat=[Pmat,P];

xhat=xhatm+K.*(zk-xhatm);

Xhat=[Xhat,xhat];

Pm=(avec.^2).*P + Q;

xhatm=avec.*xhat;

em = x(k)*[1;1] - xhatm;

ehatm = [ehatm em];

end

tvec=1:n;

figure(2)

hold

plot(tvec,Xhat(1,:),'b', tvec,Xhat(2,:),'r')

xlabel('Time (k)')

ylabel('x_hat(k)')

title('AR(1) Process(BLACK), xhat(1)(BLUE) & xhat2(2)(RED)')

pause

figure(3)

dxhat=Xhat(1,:)-Xhat(2,:);

plot(tvec,dxhat)

xlabel('Time (k)')

ylabel('del_xhat(k)')

title('Difference in the AR(1) Process Predictions')

pause

figure(4)

plot(tvec,pr(1,:),'b*',tvec,pr(2,:),'r+')

xlabel('Time (k)')

ylabel('p(k)')

title('Time-Varying Probability Estimate for a1(*) and a2 (+)')

pause

figure(5)

plot(tvec,pr(2,:))

xlabel('Time (k)')

title('Probability of detection')