Lecture 2                Some Popular Continuous Random Variables

In the last lecture we addressed a number of popular discrete random variables. In these notes we will address sompe popular continuous random variables. Recall how these types of random variables are defined.

Definition 1 Let X denote a random variable with sample space SX. If the number of elements in SXis finite or countably infinite, then X is said to be a discrete random variable. If SXis a continuum on the real line, then X is said to be a continuous random variable.

The Uniform Random Variable- A random variable X with SX=[a,b] , [a,b) , (a,b] or (a,b)and with probability density function (pdf) fX(x)=1(b−a) is said to be a uniform random variable.

[Related Matlab commands are: unifpdf, unifcdf, unifrnd]

[ https://en.wikipedia.org/wiki/Uniform_distribution_(continuous) ]

Example 1 Calibration of many instruments and modeling of many signals entail the use of sinusoids. Recall that a sinusoid signal has the form s(t)=Asin(ωt+θ). In particular, s(0)=Asin(θ). Hence, the amplitude at t=0depends on the phase variable θ. By taking repeated snapshots of this sinusoid at randomly spaced intervals (e.g. using an oscilloscope in the free-run mode), one can assume that  Θ= the act of recording the phase is a random variable that has a uniform distribution over SΘ=[−π , π). The pdf is, therefore, fΘ(θ)=12π for −π≤θ<π.

(a) Compute the corresponding cumulative distribution function (cdf).  Pr[Θ≤θ]=ΔFΘ(θ)=−πθfΘ(φ) dφ=−πθ12π dφ=θ+π2π.

(b) Compute Pr[−0.1<Θ<0.1]. Pr[−0.1<Θ<0.1] =  unifcdf(0.1,-pi,pi)-unifcdf(-0.1,-pi,pi)  = 0.0318.

(c) Simulate 3 snapshots of s(t)=sin(2t+Θ) over a 5-second time window.

%Example 1

t=0:.001:5;

nt=length(t);

nsim=3;

th=unifrnd(-pi,pi,nsim,1);

s=zeros(nsim,nt);

for k=1:nsim

s(k,:)=sin(2*t + th(k));

end

plot(t,s)

title('Snapshots of s(t)')

xlabel('Time (sec)')

grid

(d)Suppose that the scope is switched from the free-run mode to the trigger mode, and that the trigger is set to take a snapshot when s(t) crosses zero with a positive slope. There will always be a slight amount of trigger ‘jitter’. Assume that Θ~Uniform(−0.2 , 0.2). Repeat (c) for this case.

The only needed code change is: th=unifrnd(-.2,.2,nsim,1);

The Exponential Random Variable- The pdf is fX(x)=λ e−λx with SX=[0,∞).

[Related Matlab commands are: exppdf, expcdf, exprnd]

[ https://en.wikipedia.org/wiki/Exponential_distribution ]

(a)Compute the corresponding cumulative distribution function (cdf).  FX(x)=0xλe−λu du=1−e−λx.

(b)Given the event [X>x0], repeat (a).

FX |X>x0(x)=Pr[X≤x | X>x0]=Pr[X≤x ∩ X>x0]Pr[X>x0]=Pr[x0X≤x ]Pr[X>x0]=FX (x)−FX (x0)1−FX (x0)=(1−e−λx)−(1−e−λx0)1−(1−e−λx0)=e−λx0−e−λxe−λx0=1−e−λ(x−x0)=FX(x−x0)

Notice that this cdf is exactly the same as that in (a), except that now the sample space is  SX=[x0,∞). Because of this, the exponential distribution is said to be memoryless (e.g. Given that a part has survived an amount of time  x0, its failure probability is exactly the same as it was when it was new).

(c)From https://en.wikipedia.org/wiki/Exponential_distribution  copy/paste the following: (i)  μX, (ii)  σX2, and (iii) plots of the pdf for various values of  λ.

The Normal Random Variable- The pdf is fX(x)=1σ2π e−(x−μ)22σ2 with SX=(−∞,∞).

[Related Matlab commands are: normpdf, normcdf, normrnd]

[  https://en.wikipedia.org/wiki/Normal_distribution  ]

Example 1 continued. Many periodic signals can be modeled as a sum of sinusoids. Suppose that for a chosen sinusoid we have Θ~N(μ=45o , σ=3o). Note the now our units are degrees.

(e)Compute Pr[40o<Θ<50o].   Pr[40o<Θ<50o]= normcdf(50,45,3) - normcdf(40,45,3) = 0.9044

(f)Repeat (c) for this case.

The needed code change is: th=normrnd(45,3,nsim,1);

Example 2 Suppose that when a lathe cutting tool is good the diameter of any turned shaft is Dg~N(μ=2 , σ=.005), and that when the tool is bad it is Db~N(μ=2.01 , σ=.005).

(a)For each condition compute Pr[1.99<D<2.01].

Pr[1.99<Dg<2.01] =  normcdf(2.01,2,.005)-normcdf(1.99,2,.005) = 0.9545

Pr[1.99<Db<2.01] =  normcdf(2.01,2.01,.005)-normcdf(1.99,2.01,.005) = 0.5000

(b)Suppose that any shaft with diameter not in the range [1.99<d<2.01] is considered as waste, and that the expense associated with each shaft is $20. Compute the total waste cost for every 1000 shafts.

For a good cutting tool, we can expect that ~955 will be good and 45 will be waste. So the cost is ~$900.

For a bad cutting tool, we can expect that ~500 will be good and 500 will be waste. So the cost is ~$10,000.

(c)To reduce the cost associated with a bad cutting tool, you have incorporated a protocol that is: Whenever a shaft diameter exceeds 2.015 the cutting tool shall be replaced. Compute the probability that you will erroneously replace a good cutting tool.

Pr[Dg>2.015] = 1-normcdf(2.015,2,.005)  = 0.0013

(d)Compute the probability that you will not replace a bad cutting tool.

Pr[Db<2.015] = normcdf(2.015,2.01,.005)  = 0.8413

(e)In view of (c-d), comment on the effectiveness of the protocol.

The Gamma Random Variable- The pdf is too ugly to give here 

[Related Matlab commands are: gampdf, gamcdf, gamrnd]

[  https://en.wikipedia.org/wiki/Gamma_distribution ]

(a)From https://en.wikipedia.org/wiki/Gamma_distribution copy/paste the following: (i)  μX, (ii)  σX2, and (iii) plots of the pdf for various values of the shape parameter k, and the scale parameter  θ.

(b)From the plots in (a) identify the one that most resembles a bell shape (i.e. normal) pdf.   BLUE  k=7.5 ; θ=1

(c) Compute expressions for k and  θ as functions of  μXand σX2.       θ=σX2μX, hence k= μX2σX2

Example 3 A collection of 100 surface roughness measurements yielded the estimates μX=48.89; σX=14.85. The skewed nature of the histogram suggested that X could be modeled by a gamma distribution.

(a)Compute the values of the shape parameter k, and the scale parameter  θ.

θ=σX2μX= 14.85^2/48.89 = 4.5106     ;     k= μX2σX2= (48.89/14.85)^2  = 10.8389

(b)Plot the pdf over an appropriate range of x-values.

Since μX+3μX=48.89+3(14.85)≃94,

I will try a plot over 0-100.

>> x=0:.01:100;

>> f=gampdf(x,4.516,10.8389);

>> plot(x,f)

Not so good . So try 0-150. Sweet 

(c) Unlike yourself, your colleague did not look at the data histogram. He simply assumed that X was normally distributed. Compute Pr[X>μX+3σX] for his model, and compare it to the value given by yours.

HIS: 1-normcdf(48.89+3*14.85,48.89,14.85) = 0.0013 ; MINE: 1-gamcdf(48.89+3*14.85,10.8389,4.5106) = 0.0065

My probability is 5 times greater than his.