Lecture 5            Using Linear Methods for Nonlinear Prediction Models and Multiple Linear Prediction

Example 1. Very often, engineers assume that a spring has a linear force-displacement relationship; that is, F = kX. This assumption relies on the assumption that the spring will not be stretched too far (i.e. an amount that is on the order of the total length of the coils, for example). In order to obtain a model for a newly designed spring that can be used over a large range of displacements, 100 springs were selected. Each spring was subjected to a randomly chosen amount of displacement, and the resulting force was recorded. A scatter plot of the results is shown at right.

Figure 1.1 Force-displacement data for 100 springs.

(a) Consider first the following linear model:  F = k X. Notice that this model includes a slope parameter, but no force-intercept parameter. This is because physics dictates that for zero displacement there must be zero force. To obtain an estimate of the spring rate, k, we will require that Fbe an unbiased predictor of F; that is:

E(F) = E(k X) = k E(X) ≡ E(F).	(1a)

The condition (1a) results in:               k = E(F)E(X) =Δ μFμX.	(1b)

If we assume that we have no prior knowledge of the means, then we must estimate them from the data. We will use the sample means for this purpose. Thus, our estimator of k is:

k = μFμX where μF =Δ 1100j=1100FjandμX =Δ 1100j=1100Xj.

From the given data, we obtain the estimates μX=2.5075 in. and μF=33.2589 lbf. Hence, our estimate corresponding to the estimator (1c) is k=13.2638 lbf /in. The model (5.17a) for this estimate of k is shown at right.

Figure 1.2 The model F=13.2368 X.

Remark 1. We could have just as easily restricted our model to the region of the scatter plot that appears to suggest a linear relationship between X and F. At higher spring displacement, x, the linear model underestimates the associated force. Every spring has this property! When the spring coils are stretched further and further, the required force becomes higher and higher in a power-law fashion. The fact is, a spring cannot be stretched any further than the total length of the coils no matter how much force is applied.

(b) We now consider the quadratic model F=a X2+bX+c. Notice that we have chosen here to include the constant, c, expecting that the model will be good enough that we will find c ≃0. This additional parameter will allow us to use the more standard approach to arriving at the model parameter estimate. DefineY =Δ X2. Then the model becomes

F = a Y+bX+c	(2)

Condition 1:                E(F)=E(F)  (i.e. Fis an unbiased estimator for F).

This condition results in:                 E(F) = a E(Y)+bE(X)+c=E(F). 	(3a)

Using our notation for means, (3a) becomes:             c = μF−aμY−bμX.	(3b)

Condition 2:                                        Cov(F−F,X)=Cov(F−F,Y)=0.

In words, we require that the prediction error be uncorrelated with both X and Y. This gives the following two equations:

Cov(F,X)=Cov(F,X)=Cov(bX+aY+c,X)=bCov(X,X)+aCov(X,Y).	(4a)

Cov(F,Y)=Cov(F,Y)=Cov(bX+aY+c,Y)=bCov(Y,X)+aCov(Y,Y).	(4b)

Using our notation for variances and covariances, (4a-b) become:

σX2σXYσXYσY2 ba = σFXσFY.	(4c)

Clearly, the solution for the unknown parameters a and b is then:

ba = σX2σXYσXYσY2−1 σFXσFY.	(4d)

Computing Estimates of a, b, and c:

Let D=Δ(X,Y,F). Then μD=Δ(μX,μY,μF) and ΣD=ΔσX2σXYσXFσXYσY2σYFσXFσYFσF2. Now, define the arrays

Σ(X,Y)=ΔσX2σXYσXYσY2 and Ψ(X,Y)=Δ σXFσYF. Then (4d) becomes:  ba = Σ(X,Y)−1 Ψ(X,Y).	(5a)

From μD=Δ(μX,μY,μF), define the array μ(X,Y)=Δ [μX μY]. Then (3b) becomes: c = μF−μ(X,Y)ba.	(5b)

Equations (5) provide a very simple procedure for estimating the model parameters from the 100×3 data array, d:

y=x.^2;

d=[x,y,f];

C=cov(d);

Cxy=C(1:2,1:2);

Sxy=C(1:2,3);

ba=Cxy^-1*Sxy;

muhatD=mean(d);

c=muhatD(3)-muhatD(1:2)*ba;

fhat=ba(2)*y+ba(1)*x+c;

%-----------------------

%To plot prediction line use specified x-values:

xo=0:.1:5;

yo=xo.^2;

fhato=ba(2)*yo+ba(1)*xo+c;

figure(2)

plot(x,f,'*')

hold on

plot(xo,fhato,'k','LineWidth',2)

xlabel('Displacement (in.)')

ylabel('Force (lb_f)')

title('Nonlinear Spring model')                                                             Figure 1.3 F=0.98X2+10.07 X −0.06.

The model used to generate the data set wasF=X2+10X +ε.            □

Example 2  A 2-D Prediction Model Example

Consider the 2-D prediction model:      Y1Y2 = a11a12a21a22 X1X2 + b1b2 ⇒ Y=AX+B.

We have

Condition 1:                        E(Y)=AE(X)+B=E(Y) ⇒ B=μY−AμX.	(6)

Condition 2:                                        Cov(Y−Y,X)=02×2.	(7a)

From (7a): Cov(Y,X)=Cov(AX+B,X)=ACov(X,X) ⇒ A=Cov(X,X)−1Cov(Y,X).	(7b)

The Matlab code is:

% PROGRAM NAME: TwoDmodeldemo.m

A = [ 10 ,1 ; 5, 5];

B = [3 ; 1];

n = 1000;

SigmaXX_true = [1 0 ; 0 1];

MuX_true = [0 ; 20];

X = mvnrnd(MuX_true , SigmaXX_true,n);

SigmaEE_true = eye(2);

MuE_true = [0 ; 0];

E = mvnrnd(MuE_true , SigmaEE_true,n);

Y = zeros(n,2);

for k = 1:n

Y(k,:) = A*X(k,:)' + B + E(k,:)';

end

nvec = 1:n; nvec = nvec';

figure(1)

XY = [X , Y];%(X,Y) nx4 data array

plot(nvec,XY)

legend('X1','X2','Y1','Y2')

xlabel('Measurement Number')

title('Plots of X & Y')

grid

pause

%================================

CXY = cov(XY)

pause

SigmaYX = CXY(3:4,1:2)

SigmaYX_true = A*SigmaXX

SigmaXX = CXY(1:2,1:2)

SigmaXX_true

pause

Ahat = SigmaYX*SigmaXX^-1

A

pause

MuXY = mean(XY);

MuX = MuXY(1:2)';

MuY = MuXY(3:4)'

MuY_true = A*MuX_true + B

pause

Bhat = MuY - Ahat*MuX

B

%=================================

% Compute Yhat:

Yhat = zeros(n,2);

for k = 1:n

Yhat(k,:) = Ahat*X(k,:)' + Bhat;

end

figure(2)

plot(nvec,Y,'k',nvec,Yhat,'r')

title('Prediction Performance')

grid

%===================================

% Estimate SigmaEE:

Ehat = Y - Yhat;

SigmaEE = cov(Ehat)

SigmaEE_true

The data are plotted below.

Ahat = [10.08    0.96  ;   4.98    4.98]  &  A =[10     1  ;  5     5]

Bhat = [3.7329  1.4105]  &  B = [3 1]

The predictions are overlaid against the data in the plot below.

□