Lecture 6 A Primer on Random Processes

Recall that a random variable, X, is an action. If that action results in a single number, x, then X is a 1-D variable. If the action results in n numbers, (x1,x2,…,xn)', then X is an n-D variable (X1,X2,…,Xn)'.

Definition 1. A continuous random process is a collection of ‘continuous-time’ indexed random variablesX(t) | t∈(−∞,∞). A discrete random process is a collection of ‘discrete-time’ indexed random variablesX(kΔ) | k∈−∞,∞ .

Example. A thermocouple is used to monitor the temperature of a lake near a nuclear power plant. A 24-hour measurement on 13 August, 2007 is shown below.

Figure 1. Sampled thermocouple output (oF) at lake location near power plant on 13 August 2007. Sampling interval Δ=0.01 Hrs.

What we have in Figure 1 is data. Specifically, we have 2400 numbersw(k)k=124. We have a variety of actions that we can associate these numbers with:

(i) X = The act of measuring temperature at any sample time. This is a 1-D random variable, and we have 2400 measurements of it. From these, we can obtain information about X:

Figure 2. An estimate of fX(x)is shown in the plot. The estimates of μXand σXare 80.1 oF and 0.95 oF.

Based on Figure 2, we can claim that X~N(80.1 , 0.952).

(ii) Y = The act of measuring temperature at the sample time just after that of X. We now have a 2-D random variable (X,Y). We already know that X~N(80.1 , 0.952). If we ignore the relation of Y to X, then marginally, we have Y~N(80.1 , 0.952). The point of defining Y is that we desire to know if there is a relationship between successive temperature measurements. A scatter plot for (X, Y) based on the data in Figure 1 is shown below.

Figure 3. A scatter plot for (X, Y) based on the data in Figure 1.

There is clear evidence that X and Y have a positive correlation. We estimate that it as ρXY=0.88.

(iii) Let Z = The act of measuring temperature at the sample time just after that of Y. Here again, if we ignore the relation of Z to both X and Y, then Z~N(80.1 , 0.952). If we ignore the relation of Z to X, but focus on only its relation to Y, then ρYZ=0.88. Hence, the point of bringing Z into the picture is that we desire to know about the relation between Z and X. A scatter plot for (X, Y) based on the data in Figure 1 is shown below.

Figure 3. A scatter plot for (X, Z) based on the data in Figure 1.

There is clear evidence that X and Z have a positive correlation. We estimate that it as ρXZ=0.82.

(iv) Let W(kΔ) | k∈−∞,∞ denote the act of measuring temperature over any infinite collection of sample times. Then at any time kΔ ,the random variable W(kΔ), ignoring all other random variables is simply X in (i). More generally, for any time kΔ ,the random variables W(kΔ) , W((k+1)Δ) , W((k+2)Δ) , ignoring all other random variables are exactly the random variables {X, Y, Z}. Were we to continue to define more and more random variables, based on how far apart in time any two measurements are, we would arrive at the random process W(kΔ) | k∈−∞,∞ . This way of defining random variables only in terms of how far apart they are from each other in time, with no concern for the absolute value of time, is tantamount to assuming the following properties of the temperature random process:

(P1) For any sample time kΔ, W(kΔ)~N(80.1 , 0.952)

(P2) Corr[W(kΔ),W((k+n)Δ)]=ρW(nΔ), where ρW(0)=1, ρW(1)=.88, ρW(2)=.82

Definition 2. A random process X(kΔ) | k∈−∞,∞ is said to be wide sense stationary (wss) if the following two conditions hold:

(C1): For any sample time kΔ, E[X(kΔ)]=μX   and    (C2): Corr[X(kΔ),X((k+n)Δ)]=ρX(nΔ),

What we see is that:

by defining random variables only in relation to time separation (ignoring absolute time) we have forced the random process W(kΔ) | k∈−∞,∞ to be a wss process!!!

Before we address the validity of this ‘enforcement’, let’s now assume that the temperature process is, indeed, wss. We will now proceed to create a model for this process. This model will be based on the short-delay trend in the values of the autocorrelation function.

Specifically, from the above we have:

(i) X(kΔ)~N(80.1 , 0.952) ∀ k

(ii) ρXY=ρX(Δ)=0.88   and   ρX(2Δ)=0.82.

Definition 3. For a wss discrete-time random process, Xk=ΔX(kΔ), the autocorrelation function is defined as

RX(m) =Δ E(XkXk+m).	(1)

In relation to the temperature random process, we can define the related process Yk=ΔY(kΔ)via:

Xk=μX+Yk.	(2)

From (2) it should be clear that Yk=ΔY(kΔ) is a zero-mean wss random process. Substituting (2) into (1) gives:

RX(m) =Δ E(XkXk+m)=E[(μX+Yk) (μX+Yk+m)]=μX2+E(YkYk+m) = μX2+RY(m).	(3)

From (3) we see that RX(m) is simply a shifted version of RY(m) . Hence, in order to arrive at a model for Xk=ΔX(kΔ) we will develop a model for Yk=ΔY(kΔ), and then simply add μX to Yk to obtain a model for (2).

From the above, we have:

(i) Y(kΔ)~N(0, 0.952) ∀ k. In other words:      RY(0)=.952=.9025≃0.9

(ii) ρY(Δ)=0.88   and   ρX(2Δ)=0.82.

In other words:  RY(1)=RY(0)ρY(1)=.9(.88)≃.79 and RY(2)=RY(0)ρY(2)=.9(.82)≃.74

So, let’s try the ‘first order’ linear prediction model: Yk=α Yk−1. Recall, that we obtain the value for the parameter α by solving: E[(Yk−Yk)Yk−1]=0. Specifically,

E[(Yk−Yk)Yk−1]=0=E(YkYk−1−αYk−12)=RY(1)−α RY(0).  Hence,

α=RY(1)RY(0).	(3)

Furthermore, by defining                   Uk=ΔYk−Yk=Yk−αYk−1

we arrive at the model:                           Yk=αYk−1+Uk.	(4)

IF in the model (4), the error process Uk is a white noise process (i.e. the random variables are iid) then the process (4) is called an AR(1) process (i.e. an Auto-Regression process of order 1).

The Matlab command xcorr.m is designed to compute Corr[X(kΔ),X((k+n)Δ)]=ρX(nΔ). However, it is disappointing that, at this point in time, the above command must be augmented by a host of code to compute ρX(nΔ). One must be careful to first subtract the data mean, prior to using the above command. Upon doing this, for the de-meaned data in Figure 1, we obtain the figure below.

Figure 5. Computation of ρW(nΔ) obtained using the command xcorr(dw,’coef’) where dw is the de-meaned data in Figure 1.

Notice that ρW(0)=1.0is placed in the center of the plot. Hence the center time index 2401 is really lag n=0. The plot is symmetric about this point since ρW(nΔ)=ρW(−nΔ). In the lower (zoomed) plot in Figure 5, we observe the numerical values that we obtained above; namely, ρW(Δ)=ρXY=0.88 and ρW(2Δ)=ρXY=0.82.

Below is the Matlab code that was actually used to generate the time series in Figure 1.

%PROGRAM NAMES: temp.m

%lake temperature near nuclear plant for 24-hr period

t=0:.01:24-0.01;

nt = length(t);   *** simulate a length-n portion of the random process w(k) ***

dw=zeros(1,nt);

a = .9;

u=.19^.5*randn(1,nt);  Note that this loop is exactly the model: Y = aX + U

dw(1)=randn(1,1);      performed in a recursive fashion.

for k=2:nt

dw(k)=a*dw(k-1)+ u(k);

end

w=80+dw;               Note that when simulating a random process with a non-zero

figure(1)              mean, the mean is added AFTER the zero-mean process is generated.

plot(t,w)

xlabel('Time (Hrs.)')

ylabel('Temperature (F)')

pause

% Let X = act of measuring Temp at any time

mx = mean(w);

sigx=std(w);

bvec = 76.05:.1:83.5;

h = hist(w,bvec);

fx = (0.1*nt)^-1 *h;

figure(2)

bar(bvec,fx)

xlabel('Temperature, x, (F)')

ylabel('f_X (x)')

pause

% Let Y = act of measuring the next Temp after X

xy = [w(1:nt-1)' , w(2:nt)'];

figure(3)

plot(xy(:,1),xy(:,2),'*')

xlabel('Temperature (F) at any time k*delta')

ylabel('Temperature at time (k+1)*delta')

Rxy = corrcoef(xy);

rxy = Rxy(1,2);

pause

% Let Z = act of measuring the next temp after Y

xz = [w(1:nt-2)' , w(3:nt)'];

figure(4)

plot(xz(:,1),xz(:,2),'*')

xlabel('Temperature (F) at any time k*delta')

ylabel('Temperature at time (k+2)*delta')

Rxz = corrcoef(xz);

rxz = Rxz(1,2);

pause

% Compute the de-meaned autocorreltation function

dw = w - mean(w);

rdw = xcorr(dw,'coef');

figure(5)

plot(rdw,'*')

xlabel('Integer time k')