Beam Loading and Deflection Relations

http://en.wikipedia.org/wiki/Deflection_%28engineering%29

Cantilevered Beam The elastic deflection f and angle of deflection φ (in radians) in the example image, a (weightless) cantilever beam, with an end load on it, can be calculated (at the free end B) using:

fB = F·L3 / (3·E·I)     ;     φB = F·L2 / (2·E·I)

where: F = force acting on the tip of the beam; L = length of the beam (span); E = modulus of elasticity; I = area moment of inertia

Simply Supported Beam

The elastic deflection on a beam, loaded at its centre, supported by two simple supports is given by:

δ = FL3/48EI

where: δ = the deflection angle of the beam; F = force acting on the centre of the beam; L = length of the beam between the supports

Young’s Modulus of Elasticity

Epine≃1.3×106psi   ;   Eoak≃1.6×106psi   ;   Esteel≃29×106psi

Area Moment of Inertia

Uniform Crossection: I=bh312

I-Beam: I=bd312 − (b−t)h312

Application to West Balcony: Epine≃1.3×106psi  I=bh312; δB = F·L3 / (3·E·I) ; φB = F·L2 / (2·E·I)

δ=FL33EI=F(663)(12)3(1.3)(106)(1.5)(9.253) =(7.45×10−4)F ≃ 10−3F.

Hence, for a 200# load on a single 2x10 cantilevered 66”, the deflection will be about ¼”. □

Application to Living Room Balcony Cross-member Loaded @ Center: δ = FL3/48EI

Using a single 2x10 spanning 16’: δ=FL348EI=F(1923)(12)48(1.3)(106)(1.5)(9.253) ≃ 10−3F

Using doubled 2x8s: δ=FL348EI=F(1923)(12)48(1.3)(106)(3)(7.253) ≃ 1.2×10−3F